Condensed 2013 Game Manual

1 Introduction

  - What is FIRST? (you should know this)

  - Gracious Professionalism (same)

1.3 Prominent FRC Awards (for a complete list and description of awards available to Teams, please reference The FRC Administrative Manual, Section 6).

1.4 Safety: A FIRST Culture

Safety is critical in FIRST and must be observed continuously by all participants. Teams are observed and evaluated at the event. “Safety Advisors” evaluate Team safety behavior and practices at Competitions. “Referees” observe safety on the playing field as well as adherence to the game rules. “Judges” evaluate how Teams have integrated safety into their robot designs when considering the Team for technical awards. Et cetera.

1.5 Robot Design and Build Schedule

The main idea of FIRST is for each team to create a new robot each year. Don’t just modify last year’s design. Repurposing things is cool but don’t do it too much.

1.6 Game Summary

Two alliances compete on a flat, 27 x 54 ft field. Each alliance has three robots. They try to score as many discs into their goals as they can during a 2:15 long match. Goal points correspond to goal height. The match begins with a 15 second Autonomous Period; Discs scored during this period are worth 2X points. For the remainder of the match, drivers control robots and try to score as many goals as possible. The match ends with robots attempting to climb on pyramids located near the middle of the field. The robot earns points based on how high it climbs. Scoring is summarized below.

Table 1-3: Disc Points

|  | Auto | Teleop |
| --- | --- | --- |
| Low Goal | 2 | 1 |
| Middle Goal | 4 | 2 |
| High Goal | 6 | 3 |
| Pyramid Goal | NEIN!1! | 5 |

Table 1-4: Pyramid Climb Points

| Level | Points |
| --- | --- |
| 1 | 10 |
| 2 | 20 |
| 3 | 30 |

2 The Arena

2.1 Overview

Figure 2-1

Field dimensions may vary up to 4 in; Design your bot to accommodate.

2.2 The ARENA

2.2.1 The FIELD

The field is a 27 by 54 ft. carpeted area, bounded by alliance walls, feeder stations, low goals, and guardrails. The floor is covered with carpet1; 3 goals are located at each end of the field above each alliance wall. A fourth goal and 3 feeder stations are located in the corners next to each alliance wall. A pyramid is adjacent to each alliance’s autonomous line. At the top of each pyramid sits a pyramid goal. Each loading zone is a trapezoidal area bounded by 2 feeder stations, an alliance wall, a guardrail, and red and blue tape.

1 (Shaw Floors, Philadelphia Commercial, Neyland II, 20, 30753, “Ground Pepper”).

Figure 2-2

The alliance walls are 6 ½ ft. high, 18 ft. wide, and define the ends of the field. The alliance wall protects the player stations and is composed of a 3 ft. high base of diamond plate aluminum topped with a 3.5 ft. high transparent polycarbonate panel. The guardrail is a system that consists of horizontal pipes that are 20 in above the floor and supported by vertical struts mounted on a 3 in aluminum angle. A transparent polycarbonate shield is attached on the inside of the guardrail, extending from the floor to the top of the guardrail, and running the length of the guardrail. The shield is intended to help prevent robots, in whole or in part, from inadvertently exiting the field during a match.

The guardrail defines the borders of the field, except where it is bounded by the alliance wall, feeder stations or low goals. Four gates in the guardrail allow access to the field for placement and removal of robots. The gates are 38 in wide, and are closed and shielded during matches.

2.2.2 FIELD Markings

Figure 2-3: Field Markings

Three lines (red, white, and blue) span the width of the field. The white line is the center line, marked with 2 in white gaffers tape. The red and blue lines are auto lines, located 108 in to either side of the center line. These are marked with 2 in red or blue gaffers tape.

Each alliance has one loading zone that is located in front of their double feeder station. This zone is defined by a 2 in Red or blue gaffers tape line that is offset 20 in from the feeder stations.

2.2.3 The Goals

Figure 2-4: The GOALS

Each alliance’s low goal is located to the left of their opponent’s control station; the opening of the low goal is 29 in wide by 24 in tall. The bottom edge of the low goal is located 19 in from the play surface. Two mid goals and one high goal are placed above each alliance wall. The two middle goals have openings that are 54 in wide by 21 in tall. The bottom edge of the mid goal is located 88 5/8 in above the field. The high goal is 54 in wide by 12 in tall with the bottom edge of the opening located 104 1/8 in above the field surface. Damping chains2 hang from the top. These chains are 9 ½ in back from the front of the goal and spaced 6 ½ in apart.

Located atop each pyramid is a pyramid goal. The pyramid goal is a polycarbonate and metal framed “basket” that is a 23 5/8 in square that is 2 3/4 in deep. Rising out of the center of the pyramid goal is a 20 in tall, 1 1/2 in diameter post. Damping chains2 hang from the post.

2(ace hardware p/n: 5365283)

Figure 2-5: PYRAMID GOAL

2.2.4 The VISION TARGETS

Each goal has an associated vision target made of retro-reflective material (3m 8830 silver marking film). The vision targets for the low, middle, and high goals consist of a 4 in wide border of retro-reflective material around the goal opening.

An additional vision target is located on either side of the center player station, made up of a 4 in wide, 32 in tall stripe of retro-reflective material bordered by 2 in wide black gaffers tape on the left and right sides. These stripes begin 36 in above the field carpet and are 72 in apart, measured center-to-center.

The pyramid goal vision target is made up of two components. The first component is a 4in wide retro-reflective marker located on the center of each horizontal metal pole of the goal basket. The second component is retro-reflective material covering the 20 in tall, 1 ½ in diameter post in the center of the pyramid goal.

Figure 2-6: The VISION TARGETS

2.2.5 The PYRAMIDS

Figure 2-7: The PYRAMIDS

An alliance-specific pyramid is located in each half of the field. The pyramid is made of 1.5 in diameter steel tubing and powder-coated red or blue.

Base: 94” wide. Side angle: 68°. Rung spacing: 30”, 60”, and 90” off the floor.

Pyramid goal is at the peak, detailed in section 2.2.4. The pyramid has four 24” square, 11 gauge (.120”) steel bases with hook and loop tape (“hook” side) on the bottom. These bases secure the pyramids to the carpet. To prevent damage to venue floors, these metal bases are surrounded by floor protectors. These floor protectors are made from 1/8” thick hardboard. The floor protectors begin 12” outside of all the pyramid bases and continue to cover the floor underneath the pyramid (not including the bases themselves). Additional carpet covers both bases and floor protectors and is held to the carpet using 2” wide, black gaffers tape. The protectors and carpet create a 1/4 - 3/8” lip around the pyramid.

Figure 2-8: PYRAMID Floor Protectors

Teams may have to remove their ROBOT from the PYRAMID after the MATCH is over with the aid of a FIRST supplied

belay system, per G04. The belaying device acts as a brake to prevent accidentally dropped ROBOTS from causing

injuries to teams. The device attaches to the ROBOTS with two carabiners or loops of rope. Teams must supply two

attachment points for the belaying device to mount to their ROBOTS, per R10.

Only trained event staff are allowed to use the belaying device.

Figure 2-9: ROBOT Using Belay Device

page 20 / 90

2.2.6 The ALLIANCE STATIONS

Figure 2-10: The ALLIANCE STATIONS

The ALLIANCE STATIONS are located at either end of the FIELD, behind the ALLIANCE WALLS. The ALLIANCE

STATION extends 10 ft. back from the ALLIANCE WALL, and spans the entire width of the wall. The ALLIANCE

STATION includes the three (3) PLAYER STATIONS, one (1) FEEDER/GOAL STATION and one (1) double FEEDER

STATION. The STARTING LINE is marked with White 2” wide gaffers tape 4 ft. behind the ALLIANCE WALL. The

ALLIANCE STATION includes the area behind the corners of the FIELD, with the end of the ALLIANCE STATION

marked out in 2” white gaffers tape. The tape boundaries are not considered part of the bounded areas.

2.2.7 The FEEDER STATIONS

page 21 / 90

Figure 2-11: The FEEDER STATIONS

There are three (3) FEEDER STATIONS per ALLIANCE. Two (2) FEEDER STATIONS are located side by side in

one corner of the FIELD to the right of the ALLIANCE’S DRIVERS. The third FEEDER STATION is located next to

the opponent’s LOW GOAL on the opposite side of the ALLIANCE STATION.

All of the FEEDER STATIONS consist of a 44” wide shield of clear polycarbonate. In the center of the shield are

three horizontal FEEDER SLOTS, 12” wide by 3” high. These slots are located 22”, 42” and 62” from the

FIELD carpet to the bottom of the slot. Each slot has a slide attached to it on the FEEDER side of the plastic. These

slides are 8” long and are at a 30-degree incline.

2.2.8 The PLAYER STATIONS

Attached to the ALLIANCE WALL in each PLAYER STATION is an aluminum shelf to support the OPERATOR

CONSOLE for the FRC Team in that PLAYER STATION. The support shelf measures 69” wide by 12” deep.

There is a 54” long by 2” wide strip of hook-and-loop tape (“loop” side) along the center of the support shelf that

may be used to secure the OPERATOR CONSOLE to the shelf. Each setup location includes a competition cable (to

provide Ethernet connectivity) that attaches to the Ethernet Port of the OPERATOR CONSOLE. The cable provides

communications with the ROBOT via the ARENA network.

Each PLAYER STATION also includes a power adapter cable that may be used to power the Classmate laptops that

were provided to teams in the Kit of Parts starting in 2010. Emergency Stop (E-Stop) buttons for each ROBOT are

located on the left side of each PLAYER STATION shelf. ARENA components (including team number displays,

competition ARENA hardware, ALLIANCE lights, control hardware cabinets and clock displays) are also located above

the PLAYER STATIONS and below the shelf.

Once plugged in to the Field Management System (FMS) via the Ethernet cable provided, the only open ports in the

ARENA network are as follows:

A. TCP 1180: This port is typically used for camera data from the cRIO to the Driver Station (DS) when the

camera is connected to port 2 on the 8-slot cRIO (P/N: cRIO-FRC). This port is bidirectional.

B. TCP 1735: SmartDashboard, bidirectional

C. UDP 1130: Dashboard-to-ROBOT control data, directional

D. UDP 1140: ROBOT-to-Dashboard status data, directional

E. HTTP 80: Camera connected via switch on the ROBOT, bidirectional

F. HTTP 443: Camera connected via switch on the ROBOT, bidirectional

page 22 / 90

Teams may use these ports as they wish if they do not employ them as outlined above (i.e. TCP 1180 can be used to

pass data back and forth between the ROBOT and the DS if the team chooses not to use the camera on port 2).

2.2.9 The Netting

In order to protect event attendees, a tall net runs parallel to both sides of the GUARDRAIL. The net is made up of

three panels that are 10 ft. tall and 14 ft. long. The net is made from black nylon rope woven into a 5” mesh, with

vinyl trim on all four sides. The panels hang on poles that are connected to steel, free standing bases (not attached to

the GUARDRAILS). The net will be set up 12” to 36” away from the GUARDRAILS depending on venue space. The

panels will be able to be folded back to allow access to the FIELD between MATCHES.

Figure 2-12: The Netting

2.2.10 The DISCS

The DISCS are Wham-O part number 53214 (custom colored Red, White, or Blue) and have a diameter of 10 5/8”, a

height of 1 1/4”, and a weight of 180 ± 5 grams.

page 23 / 90

Figure 2-13: Red, White, and Blue DISCS

3 The Game

FIRST®, the FIRST® logo, FIRST® Robotics Competition, FRC ®, Coopertition®, and Gracious

Professionalism® are registered trademarks, and Sport for the Mind™ and ULTIMATE ASCENTSM are common

law trademarks, of the United States Foundation for Inspiration and Recognition of Science and Technology

(FIRST®). ©2013 FIRST. All rights reserved.

Teams playing ULTIMATE ASCENTSM use their ROBOTS to collect and score DISCS in a variety of GOALS.

Teams earn additional points by CLIMBING their ALLIANCE’S PYRAMID by the end of the MATCH. This

chapter contains two sections: the first section describes the game and what Teams may expect, and the

second section covers the detailed rules by which competitors must abide.

3.1 Game Details

Details such as the FIELD set up, MATCH execution, point values, etc. are defined in this section.

3.1.1 MATCH Setup

Before the start of each MATCH, the ARENA is populated with 118 White DISCS, 6 Red DISCS, and 6 Blue DISCS.

DISCS are staged as follows:

A. No more than two (2) or three (3) White DISCS may be preloaded in each ROBOT by the TEAM, depending on

the ROBOT’S starting position (see G06 for details).

B. At least ten (10) White DISCS are staged on the FIELD approximately as illustrated in Figure 3-1.

A. Two (2) DISCS are centered between the GUARDRAILS and 78” from the Blue ALLIANCE WALL to

the center of the DISC.

B. Two (2) DISCS are centered under the Blue PYRAMID.

C. At least two (2) DISCS are placed centrally between the GUARDRAILS and along the field CENTER

LINE.

Any DISCS not preloaded in ROBOTS per part Aabove (e.g. the ROBOT is preloaded with fewer than

three (3) DISCS, the ROBOT isn’t in the MATCH, etc.) are placed adjacent to the DISCS on the

CENTER LINE. The non-preloaded DISCS are added to each side of the existing pair such that the

group of DISCS on the CENTER LINE is centered. If there is an odd number of DISCS staged on the

CENTER LINE, the extra DISC will be placed on the scoring table side of the group.

D. Two (2) DISCS are centered under the Red PYRAMID.

E. Two (2) DISCS are centered between the GUARDRAILS and 78” from the Red ALLIANCE WALL to

the center of the DISC.

C. 6 Red & 45 White DISCS are located in the Red ALLIANCE STATION.

D. 6 Blue & 45 White DISCS are located in the Blue ALLIANCE STATION.

Figure 3-1: Starting Locations for DISCS

3.1.2 MATCH Timing

A MATCH is two (2) minutes and fifteen (15) seconds long. The Autonomous (AUTO) Period is the first fifteen (15)

seconds of the MATCH. The Teleoperated Period (TELEOP) is the remaining two (2) minutes. Each Period ends when

the ARENA timer displays zero (0) seconds.

3.1.3 MATCH Logistics

DISCS that are ejected from gameplay during a MATCH will be placed back in the FIELD by event staff approximately

where they left and at the next safe opportunity.

3.1.4 Penalty Assignment

page 25 / 90

Upon a rule violation, FOUL or TECHNICAL FOUL points will immediately be credited to the opposing ALLIANCE.

Values are defined in Table 3-1.

Table 3-1: Penalty Point Values

FOUL 3

TECHNICAL FOUL 20

3.1.5 Scoring

Points are awarded to ALLIANCES per the details below. Final scores will be assessed five (5) seconds after the

ARENA timer displays zero (0) or when all elements come to rest, whichever event happens first.

3.1.5.1 DISC Points

Points are awarded for DISCS SCORED in the GOALS per Table 3-2.

A DISC is considered SCORED in an ALLIANCE’S GOAL if any part of the DISC has crossed through the opening of

the GOAL, is in the GOAL at the end of the MATCH, and is not in contact with any ROBOT from that ALLIANCE.

GOAL openings are outlined in yellow in Figure 3-2 and Figure 3-3.

Figure 3-2: LOW, MIDDLE, and HIGH GOAL openings

Additionally, for a DISC to be considered SCORED in an ALLIANCE’S PYRAMID GOAL, it must correspond to the

PYRAMID color.

page 26 / 90

Figure 3-3: PYRAMID GOAL opening

Table 3-2: DISC Point Values

AUTO TELEOP

LOW GOAL 2 1

MIDDLE GOAL 4 2

HIGH GOAL 6 3

PYRAMID GOAL N/A 5

3.1.5.2 CLIMB Points

Points are awarded for the highest Level achieved for every ALLIANCE ROBOT that CLIMBS its PYRAMID. The Level

to which a ROBOT has CLIMBED is determined by the lowest point of the ROBOT (in relation to the FIELD). CLIMB

point values and Levels are defined in Figure 3-4.

page 27 / 90

Figure 3-4: PYRAMID Levels

As competition at the FIRST Championship is typically different from that during the

competition season, FIRST may alter the value of CLIMBING at the FIRST

Championship by up to ten (10) points per Level.

A ROBOT has CLIMBED its PYRAMID if it contacts the PYRAMID in

A. sequential order (Level 0, 1, 2, 3) during ascent and

B. no more than two (2) Levels simultaneously.

If a CLIMB is considered unacceptable (e.g. a ROBOT has touched non-adjacent Levels or more than two (2) Levels at

a time), a Referee will indicate a rejected CLIMB by turning the offending ROBOT’S PLAYER STATION LED strings

yellow. The ROBOT will be ineligible for CLIMB points unless and until it begins a new CLIMB from the floor, Level 0.

page 28 / 90

3.2 Game Rules

The rules in this section legislate game play and define the consequences for rule violations.

3.2.1 Safety

3.2.1.1 G01

Teams may only enter the FIELD if the PLAYER STATION LED strings are green.

Violation: YELLOW CARD

3.2.1.2 G02

Teams may not climb on the PYRAMID.

Violation: YELLOW CARD

For the purpose of G02, this is the colloquial use of “climb,” not the ULTIMATE ASCENT

use of “CLIMB.”

3.2.1.3 G03

ROBOTS whose operation or design is unsafe are not permitted.

Violation: FOUL & DISABLED. If the issue is due to design: Re-Inspection.

An example of unsafe operation would be uncontrolled motion that cannot be stopped by

the DRIVERS.

3.2.1.4 G04

ROBOTS may only be removed from a PYRAMID under the following conditions:

A. by the TEAM,

B. unpowered,

C. and under the supervision of a FIRST Technical Advisor (FTA), FTA Assistant, Referee, or Field Supervisor.

page 29 / 90

Additionally, if any part of the ROBOT is in Level 3, TEAMS are required to attach a FIRST supplied belay line, detailed

in Section 2.2.6, to their ROBOT to spot a ROBOT while the TEAM removes it from the PYRAMID.

Violation: YELLOW CARD

3.2.2 Pre-MATCH

3.2.2.1 G05

When placed on the FIELD, each ROBOT must be:

A. in compliance with all ROBOT rules (i.e. have passed Inspection),

B. confined to its STARTING CONFIGURATION,

C. fully supported by the floor, and

D. contacting its PYRAMID.

Violation: If fix is a quick remedy: the MATCH won’t start until all requirements are met. If it is not a quick remedy: the

ROBOT will be DISABLED and must be re-Inspected.

3.2.2.2 G06

TEAMS may preload White DISCS in or on the ROBOT before the MATCH.

A. If the ROBOT is in contact with any carpet outside its AUTO ZONE, the ROBOT may preload up to three (3)

DISCS.

B. Otherwise, the ROBOT may preload up to two (2) DISCS.

Violation: If the situation is not corrected before the start of the MATCH, TECHNICAL FOUL per extra preloaded DISC

in or on the ROBOT.

3.2.2.3 G07

TEAMS may not cause significant or repeated delays to the start of a MATCH.

Violation: ROBOT will be DISABLED.

TEAMS are expected to stage their ROBOTS for a MATCH safely and swiftly. TEAM

efforts that, either intentionally or unintentionally, delay the start of a MATCH will not be

tolerated. Examples of such delays include, but are not limited to:

page 30 / 90

A. Use of alignment devices such as templates, tape measures, laser pointers, etc. to

precisely place and/or align the ROBOT.

B. Late arrival to the FIELD.

C. Being indecisive about where/how to position a ROBOT.

D. Installing BUMPERS, or any ROBOT maintenance or assembly, once on the FIELD.

3.2.2.4 G08

TEAMS may not leave items other than ROBOTS on the FIELD prior to or during the MATCH.

Violation: The MATCH will not start until the situation is corrected.

3.2.2.5 G09

Each TEAM member must be:

A. in the ALLIANCE STATION and

B. behind the STARTING LINE.

Violation: MATCH will not start until the situation is corrected.

3.2.3 General Rules

3.2.3.1 G10

Only TEAM members and their ROBOT may report to the ARENA for a MATCH. TEAM members are limited to:

A. 1 COACH,

B. 2 DRIVERS, and

C. 1 FEEDER

Violation: MATCH will not start until the situation is corrected.

3.2.3.2 G11

The COACH must wear the designated “COACH” button while in the ARENA.

page 31 / 90

Violation: MATCH will not start until the situation is corrected.

3.2.3.3 G12

ROBOTS may not contact anything outside the FIELD.

Violation: ROBOT will be DISABLED. However, if it occurs during AUTO, and there is no safety concern, the Head

Referee will allow a 10-second grace period at the beginning of TELEOP for the ROBOT to correct the situation.

3.2.3.4 G13

ROBOTS may not intentionally detach or leave parts on the FIELD.

Violation: TECHNICAL FOUL

3.2.3.5 G14

The following actions are prohibited with regards to interaction with FIELD elements:

A. grabbing,

B. grasping

C. grappling

D. attaching to,

E. damaging,

F. becoming entangled

Actions A-D do not apply to ROBOT interactions with the PYRAMID.

Violation: FOUL. If the Head Referee determines that further damage is likely to occur, DISABLED. Corrective action

(such as eliminating sharp edges, removing the damaging mechanism, and/or re-Inspection) may be required before

The robot will be allowed to compete in subsequent matches.

Robots may push or react against any element of the field.

Discs are expected to undergo a reasonable amount of wear and tear as they are

Handled by robots, such as scratches and occasional marks. Robots that gouge,

Tear off pieces, or routinely mark discs will be considered in violation of g14.

3.2.3.6 g15

Robots may not intentionally eject discs from gameplay.

Page 32 / 90

Violation: foul per disc.

3.2.3.7 g16

Teams and/or robots may not employ strategies that use discs to either aid or inhibit a robot climb.

Violation: technical foul. If the disc(s) inhibits an opponent’s climb attempt, the opponent robot’s

Alliance is awarded points for a successful level 3 climb.

3.2.3.8 g17

An alliance may not put discs in their opponents’ pyramid goal.

Violation: technical foul per disc.

3.2.3.9 g18

All Teams must be civil towards other Teams, competition personnel, and event attendees.

Violation: Potential RED CARD for violations in the ARENA.

Teams will not receive RED/YELLOW CARDS for off-ARENA actions; however,

designated competition personnel will hold them accountable for their off-ARENA actions.

3.2.4 AUTO Rules

3.2.4.1 G19

During AUTO, a ROBOT may not cross the CENTER LINE such that it is no longer in contact with the carpet on its

starting half of the FIELD.

Violation: FOUL. If contact with an opponent ROBOT, TECHNICAL FOUL.

3.2.4.2 G20

During AUTO, TEAMS must remain behind the STARTING LINE.

Violation: FOUL. If contact with the OPERATOR CONSOLE, TECHNICAL FOUL.

page 33 / 90

Exceptions will be made for person or equipment safety situations (e.g. catching a falling

OPERATOR CONSOLE).

3.2.4.3 G21

During AUTO, any control devices worn or held by the DRIVERS must be disconnected from the OPERATOR

CONSOLE and not connected until TELEOP.

Violation: FOUL

3.2.5 ROBOT Actions

3.2.5.1 G22

ROBOT height (as defined in relation to the ROBOT) must be restricted as follows during the MATCH:

A. If in contact with the carpet in its AUTO ZONE and/or its PYRAMID, ? 84”

B. Otherwise, ? 60”

Violation: FOUL. If continuous or repeated violations, TECHNICAL FOUL.

3.2.5.2 G23

A ROBOT’S horizontal dimensions may never exceed a 54” diameter vertical cylinder.

Violation: FOUL. If continuous or repeated violations, TECHNICAL FOUL.

In other words, a ROBOT must always fit inside a cylinder with a diameter of 54” This

method for restricting ROBOT size requires extra diligence if a ROBOT’S geometric

center shifts as various appendages are extended and retracted.

page 34 / 90

Figure 3-5: ROBOT inside 54” diameter cylinder

3.2.5.3 G24

ROBOTS may not actively control more than four (4) DISCS at any one time.

Violation: FOUL per extra DISC

Moving or positioning a DISC to gain advantage is considered “active control.”

Examples of “active control” include

A. “carrying” (holding DISCS in or on the ROBOT),

B. “herding” (intentionally pushing or impelling DISCS to a desired location or direction),

and

C. “trapping” (pressing one or more DISCS against a FIELD element in an attempt to

shield them).

Examples of DISC interaction that are not “active control” are

D. “bulldozing” (inadvertently coming in contact with DISCS that happen to be in the path

of the ROBOT as it moves about the FIELD) and

E. “deflecting” (being hit by a propelled DISC that bounces or rolls off the ROBOT).

A DISC that becomes unintentionally lodged on a ROBOT will be considered actively

controlled by the ROBOT. It is important to design your ROBOT so that it is impossible to

inadvertently or intentionally control more than four (4) DISCS at a time.

3.2.5.4 G25

page 35 / 90

ROBOTS on the same ALLIANCE may not blockade the FIELD in an attempt to stop the flow of the MATCH. This rule

has no effect on individual ROBOT-ROBOT interaction.

Violation: TECHNICAL FOUL

3.2.5.5 G26

ROBOTS may not intentionally fall down or tip over to block the FIELD.

Violation: TECHNICAL FOUL

3.2.5.6 G27

ROBOTS may not contact or otherwise interfere with their opponents’ PYRAMID. Inconsequential contact will not be

penalized.

Violation: TECHNICAL FOUL. If an opponent’s CLIMB is affected,

A. RED CARD, and

B. Each affected opponent ROBOT will be awarded points for a successful Level 3 CLIMB.

3.2.6 ROBOT-ROBOT Interaction

3.2.6.1 G28

Strategies aimed at the destruction or inhibition of ROBOTS via attachment, damage, tipping, or entanglement of

ROBOTS are not in the spirit of the FRC and are not allowed.

Violation: TECHNICAL FOUL and YELLOW CARD

For example, use of wedge-like MECHANISM to flip ROBOTS would be considered a

violation of G28.

3.2.6.2 G29

Deliberate or damaging contact with an opponent ROBOT on or inside its FRAME PERIMETER is not allowed.

Violation: TECHNICAL FOUL

page 36 / 90

High speed accidental collisions may occur during the MATCH and are expected.

ROBOTS extend elements outside of the FRAME PERIMETER at their own risk; no

penalties will be assigned for contact between two such extended elements.

A ROBOT with an element outside its FRAME PERIMETER may be penalized under this

rule if it appears they are using that element to purposefully contact another ROBOT

inside its FRAME PERIMETER. Regardless of intent, a ROBOT with an element outside

its FRAME PERIMETER that causes damage to another ROBOT inside of its FRAME

PERIMETER will be penalized.

3.2.6.3 G30

Regardless of who initiates the contact, a ROBOT may not contact an opponent ROBOT

A. contacting its PYRAMID or

B. touching the carpet in its LOADING ZONE.

Violation: FOUL. If purposeful or consequential, TECHNICAL FOUL.

3.2.6.4 G31

An ALLIANCE may not pin an opponent ROBOT for more than five (5) seconds. A ROBOT will be considered pinned

until the ROBOTS have separated by at least six (6) ft. The pinning ROBOT(S) must then wait for at least three (3)

seconds before attempting to pin the same ROBOT again Pinning is transitory through other objects.

Violation: TECHNICAL FOUL

If the pinned ROBOT chases the pinning ROBOT upon retreat, the pinning ROBOT will

not be penalized per G31, and the pin will be considered complete.

3.2.6.5 G32

Fallen (i.e. tipped over) ROBOTS attempting to right themselves (either by themselves or with assistance from an

ALLIANCE partner) have one (1) ten (10)-second grace period per fallen ROBOT in which the fallen ROBOT may not

be contacted by an opposing ROBOT.

This protection lasts for either ten (10) seconds or until the protected ROBOT has completed the righting operation,

whichever comes first.

Violation: If inadvertent, FOUL. If intentional, TECHNICAL FOUL.

page 37 / 90

Once the 10-second grace period for righting a fallen ROBOT has expired, opposing

ROBOTS may interact with a fallen ROBOT with no FOUL assessed as long as G28 is

not violated (as applied to the fallen over ROBOT).

3.2.6.6 G33

A ROBOT may only be supported (fully or partially) by another ROBOT if one of the ROBOTS is in contact with a

PYRAMID.

Violation: If extended, strategic, or repeated, TECHNICAL FOUL.

3.2.7 Human Actions

3.2.7.1 G34

Only FEEDERS may touch DISCS. Inadvertent or inconsequential contact by others will not be penalized.

Violation: FOUL

3.2.7.2 G35

DISCS may be fed onto the FIELD only under the following circumstances:

A. during TELEOP through the FEEDER SLOTS and

B. during the last thirty (30) seconds of TELEOP over the FEEDER STATIONS.

Violation: FOUL

3.2.7.3 G36

During the MATCH, TEAMS must be within their ALLIANCE STATION. Exceptions will be granted for inadvertent or

inconsequential infractions and in cases concerning safety.

Violation: FOUL

3.2.7.4 G37

TEAMS may not extend any body part into the FIELD or contact any ROBOT at any time during the MATCH.

page 38 / 90

Violation: TECHNICAL FOUL

3.2.7.5 G38

During a MATCH, the ROBOT shall be operated solely by the DRIVERS of that TEAM.

Violation: TECHNICAL FOUL

Exceptions may be made before a MATCH for major conflicts, e.g. religious holidays,

major testing, transportation issues, etc.

4 The Robot

FIRST®, the FIRST® logo, FIRST® Robotics Competition, FRC ®, Coopertition®, and Gracious

Professionalism® are registered trademarks, and Sport for the Mind™ and ULTIMATE ASCENTSM are common

law trademarks, of the United States Foundation for Inspiration and Recognition of Science and Technology

(FIRST®). ©2013 FIRST. All rights reserved.

This section of the 2013 FRC® Game Manual presents legislation relevant to the construction of a 2013 FIRST®

Robotics Competition (FRC) ROBOT. ROBOTS will be Inspected at each FRC event to confirm compliance

before being allowed to compete, per Section 5.5.2 in The Tournament, Eligibility and Inspection of the 2013 FRC

Game Manual.

4.1 ROBOT Rules

The rules listed below explicitly address what and how parts and materials may be used on a 2013 FRC ROBOT.

There are many reasons for the structure of the rules, including safety, reliability, parity, creation of a reasonable

design challenge, adherence to professional standards, impact on the competition, compatibility with the Kit of Parts

(KOP), etc. When reading these rules, please use technical common sense (engineering thinking) rather than

“lawyering” the interpretation and splitting hairs over the precise wording in an attempt to find loopholes. Try to

understand the reasoning behind a rule.

In addition, another intent of these rules is to have all energy sources and active actuation systems on the ROBOT

(e.g. batteries, compressors, motors, servos, cylinders, and their controllers) drawn from a well-defined set of options.

This is to ensure that all Teams have access to the same actuation resources, and to ensure that the Inspectors are

able to accurately assess the legality of a given part.

Teams may be asked to provide documentation proving legality of non-2013 KOP items during Inspection where a

page 39 / 90

Rule specifies limits for a legal part (e.g. pneumatic components, current limits, COTS electronics, etc.).

Some of these rules make use of English unit requirements for parts. If your team has a question about a

metric-equivalent part’s legality, please e-mail your question to frcparts@usfirst.org for an official ruling. To seek

approval for alternate devices for inclusion in future FRC seasons, please contact frcparts@usfirst.org with item

specifications.

Teams should acknowledge the support provided by the corporate Sponsors and Mentors with an appropriate display

of their school and Sponsors names and logos (or the name of the supporting youth organization, if appropriate).

4.1.1 General ROBOT Design

4.1.1.1 R01

Each registered FRC team may enter only one (1) ROBOT into the 2013 FRC. The ROBOT must be built by the FRC

Team to perform specific tasks when competing in ULTIMATE ASCENT. The ROBOT must include all of the basic

systems required to be an active participant in the game – power, communications, control, mobility, and actuation.

The ROBOT implementation must obviously follow a design approach intended to play ULTIMATE ASCENT (e.g. a

box of unassembled parts placed on the FIELD, or a ROBOT designed to play a different game would not satisfy this

definition).

4.1.1.2 R02

The ROBOT must have a FRAME PERIMETER, contained within the BUMPER ZONE, that is comprised of fixed,

non-articulated structural elements of the ROBOT. Minor protrusions no greater than ¼ in such as bolt heads, fastener

ends, and rivets are not considered part of the FRAME PERIMETER.

To determine the FRAME PERIMETER, wrap a piece of string around the ROBOT at the

BUMPER ZONE described in R25. The string describes this polygon.

Note: to permit a simplified definition of the FRAME PERIMETER and encourage a tight,

robust connection between the BUMPERS and the FRAME PERIMETER, minor

protrusions such as bolt heads, fastener ends, rivets, etc. are excluded from the

determination of the FRAME PERIMETER.

4.1.1.3 R03

The ROBOT must satisfy the following size constraints:

A. total length of the FRAME PERIMETER sides may not exceed 112 in (see Figure 4-1 for examples),

B. PLAYING CONFIGURATION horizontal dimensions may never exceed a 54 in diameter vertical cylinder (see

G23), and

C. height may never exceed 84 in tall.

page 40 / 90

Figure 4-1: FRAME PERIMETER Length Calculations

Consider G22 restricting ROBOT height on various parts of the FIELD during the

MATCH when designing and building the ROBOT.

Expect to have to demonstrate a ROBOT’S ability to constrain itself per B above during

Inspection. PLAYING CONFIGURATION constraints may be implemented with either

hardware or software.

4.1.1.4 R04

In the STARTING CONFIGURATION, no part of the ROBOT may extend outside the vertical projection of the FRAME

PERIMETER, with the exception of minor protrusions such as bolt heads, fastener ends, rivets, etc.

If a ROBOT is designed as intended and pushed up against a vertical wall (in STARTING

CONFIGURATION and with BUMPERS removed), only the FRAME PERIMETER (or

minor protrusions) will be in contact with the wall.

4.1.1.5 R05

The ROBOT weight may not exceed 120 lbs. When determining weight, the basic ROBOT structure and all elements of

all additional MECHANISMS that might be used in different configurations of the ROBOT shall be weighed together.

For the purposes of determining compliance with the weight and volume limitations, the items listed below are not

included in the weight assessment:

A. the ROBOT battery and its associated half of the Anderson cable quick connect/disconnect pair (including no

more than 12 in of cable per leg, the associated cable lugs, connecting bolts, and insulation) and

B. BUMPERS (including BUMPER covers, if appropriate).

4.1.1.6 R06

page 41 / 90

Traction devices may not have surface features such as metal, sandpaper, hard plastic studs, cleats, or similar

attachments. Traction devices include all parts of the ROBOT that are designed to transmit any propulsive and/or

braking forces between the ROBOT and FIELD carpet.

4.1.1.7 R07

ROBOTS must allow removal of DISCS from the ROBOT and the ROBOT from FIELD elements while disabled and

powered off.

ROBOTS will not be re-enabled after the MATCH, so Teams must be sure that DISCS

and ROBOTS can quickly, simply, and safely be removed. Teams may be asked to

demonstrate this during Inspection.

4.1.2 Safety & Damage Prevention

4.1.2.1 R08

ROBOT parts shall not be made from hazardous materials, be unsafe, cause an unsafe condition, or interfere with the

operation of other ROBOTS.

Examples of items that will violate R08 include (but are not limited to):

A. Shields, curtains, or any other devices or materials designed or used to obstruct or

limit the vision of any drivers and/or coaches and/or interfere with their ability to safely

control their ROBOT

B. Speakers, sirens, air horns, or other audio devices that generate sound at a level

sufficient to be a distraction

C. Any devices or decorations specifically intended to jam or interfere with the remote

sensing capabilities of another ROBOT, including vision systems, acoustic range finders,

sonars, infrared proximity detectors, etc. (e.g. including imagery on your ROBOT that, to

a reasonably astute observer, mimics the VISION TARGET)

D. Exposed lasers other than Class I.

E. Flammable gasses

F. Any device intended to produce flames or pyrotechnics

G. Hydraulic fluids or hydraulic components

Teams should provide MSD Sheets for any materials they use that might be considered

questionable during ROBOT Inspection.

page 42 / 90

4.1.2.2 R09

Protrusions from the ROBOT and exposed surfaces on the ROBOT shall not pose hazards to the ARENA elements or

people.

If the ROBOT includes protrusions that form the “leading edge” of the ROBOT as it

drives and have a surface area of less than 1 in2, it will invite detailed Inspection. For

example, forklifts, lifting arms, or grapplers may be carefully Inspected for these hazards.

4.1.2.3 R10

Teams must supply at least two (2) attachment points for the belaying device (see Section 2.2.5) to mount to their

ROBOTS. Attachment points must be:

A. easily accessible after the ROBOT has CLIMBED the PYRAMID,

B. on opposite sides of the ROBOT,

C. located near the ROBOT’S balance point, and

D. made from exposed structural members that will allow a rope to be wrapped around it or two eyelets (McMaster

PN3014T45 or similar) mounted to the frame. Opening of the eyelets must be at least ¾ in in diameter.

4.1.3 Budget Constraints

4.1.3.1 R11

The total cost of all items on the ROBOT shall not exceed $4000 USD. All costs are to be determined as explained in

Section 4.1.3: Budget Constraints. Exceptions are as follows:

A. individual fasteners, adhesives, and lubricants, that are less than $1 each and

B. Kit of Parts (KOP) items

Teams should be prepared to disclose to Inspectors the cost of any non-KOP item and

the total cost of the ROBOT.

Per T7, teams must be prepared to display a Bill of Materials (BOM) to Inspectors during

Inspection. The BOM may be displayed in either printed or electronic form.

4.1.3.2 R12

No individual item shall have a value that exceeds $400 USD. The total cost of COMPONENTS purchased in bulk may

exceed $400 as long as the cost of an individual COMPONENT does not exceed $400.

page 43 / 90

4.1.3.3 R13

Individual COMPONENTS or MECHANISMS, not excluded in R11, that are retrieved from previous ROBOTS and used

on 2013 ROBOTS must have their undepreciated cost included in the 2013 ROBOT BOM and applied to the overall

cost assessment.

4.1.3.4 R14

The BOM cost of each non-KOP item must be calculated based on the unit fair market value for the material and/or

labor, except for labor provided by team members (including sponsor employees who are members of the team) and

shipping.

Example: A Team orders a custom bracket made by a company to the Team's

specification. The company’s material cost and normally charged labor rate apply.

Example: A Team receives a donated sensor. The company would normally sell this

item for $52, which is therefore its fair market value.

Example: Special price discounts from National Instruments and other FRC Suppliers

are being offered to all FIRST Teams. The discounted purchase price of items from

these sources may be used in the additional parts accounting calculations.

Example: A Team purchases steel bar stock for $10 and has it machined by a local

machine shop. The machine shop is not considered a team Sponsor, but donates two

(2) hours of expended labor anyway. The Team must include the estimated normal cost

of the labor as if it were paid to the machine shop, and add it to the $10.

Example: A Team purchases steel bar stock for $10 and has it machined by a local

machine shop that is a recognized Sponsor of the Team. If the machinists are

considered members of the Team, their labor costs do not apply. The total applicable

cost for the part would be $10.

It is in the best interests of the Teams and FIRST to form relationships with as many

organizations as possible. Teams are encouraged to be expansive in recruiting and

including organizations in their team, as that exposes more people and organizations to

FIRST. Recognizing supporting companies as Sponsors of, and members in, the Team

is encouraged, even if the involvement of the Sponsor is solely through the donation of

fabrication labor.

Example: A Team purchases a 4 by 4 ft sheet of aluminum, but only uses a piece 10 by

10 in on their ROBOT. The Team identifies a source that sells aluminum sheet in 1 by 1

ft pieces. The Team may cost their part on the basis of a 1 by 1 ft piece, even though

they cut the piece from a larger bulk purchase. They do not have to account for the

entire 4 by 4 ft bulk purchase item.

4.1.3.5 R15

page 44 / 90

If a COTS item is part of a modular system that can be assembled in several possible configurations, then each

individual module must fit within the price constraints defined in R12.

If the modules are designed to assemble into a single configuration, and the assembly is functional in only that

configuration, then the total cost of the complete assembly including all modules must fit within the price constraints

defined in R12.

In summary, if a VENDOR sells a system or a kit, a team must use the entire system/kit

Fair Market Value and not the value of its COMPONENT pieces.

Example1: VENDOR A sells a gearbox that can be used with a number of different gear

sets, and can mate with two different motors they sell. A team purchases the gearbox, a

gear set, and a motor (which are not offered together as an assembly or kit), then

assembles them together. Each part is treated separately for the purpose of BOM

costing, since the purchased pieces can each be used in various configurations.

Example2: VENDOR B sells a robotic arm assembly that the team wants to use.

However, it costs $700, so they cannot use it. The Vendor sells the “hand”, “wrist” and

“arm” as separate assemblies, for $200 each. A team wishes to purchase the three

components separately, then reassemble them. This would not be legal, as they are

really buying and using the entire assembly, which has a Fair Market Value of $700.

4.1.4 Fabrication Schedule

4.1.4.1 R16

ROBOT elements, including software, that are designed or created before Kickoff are not permitted, unless they are

publicly available prior to Kickoff.

Please note that this means that FABRICATED ITEMS from ROBOTS entered in

previous FIRST competitions may not be used on ROBOTS in the 2013 FRC. Before the

formal start of the FRC Build Season, Teams are encouraged to think as much as they

please about their ROBOTS. They may develop prototypes, create proof-of-concept

models, and conduct design exercises. Teams may gather all the raw stock materials

and COTS COMPONENTS they want.

Example 1: A Team designs and builds a two-speed shifting transmission during the fall

as a training exercise. After Kickoff, they utilize all the design principles they learned in

the fall to design their ROBOT. To optimize the transmission design for their ROBOT,

they improve the transmission gear ratios and reduce the size, and build two new

transmissions, and place them on the ROBOT. All parts of this process are permitted

activities.

Example 2: The same Team realizes that the transmission designed and built in the fall

perfectly fits their need for a transmission to drive the ROBOT arm. They build an exact

copy of the transmission from the original design plans, and bolt it to the ROBOT. This

would be prohibited, as the transmission – although fabricated during the competition

season – was built from detailed designs developed prior to Kickoff.

page 45 / 90

Example 3: A Team developed an omni-directional drive system for the 2011

competition. Over the summer of 2011 they refined and improved the control software

(written in C) to add more precision and capabilities. They decided to use a similar

system for the 2013 competition. They copied large sections of unmodified code over

into the control software of the new ROBOT (also written in C). This would be a violation

of the schedule constraint, and would not be allowed.

Example 4: The same Team decides to use LabVIEW as their software environment for

2013. Following Kickoff, they use the previously-developed C code as a reference for

the algorithms and calculations required to implement their omni-directional control

solution. Because they developed new LabVIEW code as they ported over their

algorithms, this would be permitted.

Example 5: A different Team develops a similar solution during the fall, and plans to use

the developed software on their competition ROBOT. After completing the software, they

post it in a generally accessible public forum and make the code available to all Teams.

Because they have made their software publicly available before Kickoff, they can use it

on their ROBOT.

4.1.4.2 R17

The ROBOT (including items intended for use during the competition in alternative configurations of the ROBOT,

excluding items permitted per R21) must be bagged or crated (as appropriate for your event), and out of Team hands

by Stop Build Day, February 19, 2013 (refer to the FRC Administrative Manual, Section 5 for more details).

4.1.4.3 R18

Teams must stay “hands-off” their ROBOT during the following time periods:

A. from Stop Build Day until their first event,

B. during the period(s) between their events, and

C. outside of Pit hours while attending events.

Additional time is allowed as follows:

D. There are no restrictions on when software may be developed.

E. On days a team is not attending an event, they may continue development of any items permitted per R21, but

must do so without interfacing with the ROBOT.

Teams attending 2-day events may access their ROBOTS per the rules defined in the Administrative Manual, Section

5.6, ROBOT Access Period - for Teams Attending 2-Day Events.

4.1.5 Material Utilization

page 46 / 90

4.1.5.1 R19

COTS items from ROBOTS entered in previous FRC competitions that are no longer commercially available may be

used only if they are functionally equivalent to the original condition as delivered from the VENDOR.

For example, a part that has non-functional label markings added would be permitted, but

a part that has device-specific mounting holes added would be prohibited.

4.1.5.2 R20

Lubricants may be used only to reduce friction within the ROBOT. Lubricants may not contaminate the ARENA or other

ROBOTS.

4.1.5.3 R21

Teams may bring a maximum of 30 lbs of FABRICATED ITEMS to each event to be used to repair and/or upgrade

their ROBOT. The OPERATOR CONSOLE, BUMPERS, and any ROBOT battery assemblies (as described in R5-A)

are exempt from this limit.

4.1.6 BUMPER Rules

4.1.6.1 R22

ROBOTS are required to use BUMPERS to protect all outside corners of the FRAME PERIMETER. For adequate

protection, at least 8 in of BUMPER must be placed on each side of each outside corner (see Figure 4-2).

The dimension defined in R22 is measured along the FRAME PERIMETER. The portion

of the BUMPER that extends into the corner is not included in the 8 in requirement.

page 47 / 90

Figure 4-2: BUMPER Corner Examples

4.1.6.2 R23

Each set of BUMPERS (including any fasteners and/or structures that attach them to the ROBOT) must weigh no more

than 20 lbs.

If a multi-part attachment system is utilized (e.g. interlocking brackets on the ROBOT and

the BUMPER), then the elements permanently attached to the ROBOT will be considered

part of the ROBOT, and the elements attached to the BUMPERS will be considered part

of the BUMPER. Each element must satisfy all applicable rules for the relevant system.

4.1.6.3 R24

BUMPERS must be constructed as follows (see Figure 4-4):

A. be backed by ¾ in (nominal) thick by 5 in (± ½ in) tall plywood or solid, robust wood.

Particle board or chipboard is not likely to survive the rigors of FRC gameplay and thus

not compliant with R24-A.

B. hard BUMPER parts (e.g. plywood, fasteners, etc) may not extend more than 1 in beyond the end of the

FRAME PERIMETER (see Figure 4-3and Figure 4-4).

page 48 / 90

Figure 4-3: Hard Parts of BUMPER Corners

C. use a stacked pair of approximately 2 ½ in round, petal, or hex “pool noodles” (solid or hollow) as the

BUMPER cushion material (see Figure 4-4). Cushion material may extend up to 2 ½ in beyond the end of the

plywood (see Figure 4-2).

D. be covered with a rugged, smooth cloth.

Silk or bedding are not considered rugged materials. 1000D Cordura is recommended.

The cloth must completely enclose all exterior surfaces of the wood and pool noodle material when the BUMPER is

installed on the ROBOT. The fabric covering the BUMPERS must be a solid Red or Blue in color. The only markings

permitted on the BUMPER fabric cover are the Team number (see Rule R31).

Visually, the Red or Blue must be as close to the corresponding color in the FIRST logo

as reasonable (i.e. to a reasonably astute observer, they appear similar).

E. must attach to the FRAME PERIMETER of the ROBOT with a rigid fastening system to form a tight, robust

connection to the main structure/frame (e.g. not attached with hook-and-loop or tie-wraps). The attachment

system must be designed to withstand vigorous game play. All removable fasteners (e.g. bolts, locking pins,

pip-pins, etc.) will be considered part of the BUMPERS.

page 49 / 90

Figure 4-4: BUMPER Cross Section

4.1.6.4 R25

BUMPERS must be located entirely within the BUMPER ZONE, which is between 2 and 10 in from the floor, in

reference to the ROBOT standing normally on a flat floor.

4.1.6.5 R26

BUMPERS may not be articulated (relative to the FRAME PERIMETER).

4.1.6.6 R27

Corner joints between BUMPERS must be filled with pool noodle material. Examples of implementation are shown in

Figure 4-5.

page 50 / 90

Figure 4-5: Soft Parts of BUMPER Corners

4.1.6.7 R28

BUMPERS (the entire BUMPER, not just the cover) must be designed for quick and easy installation and removal.

As a guideline, BUMPERS should be removable by two (2) people in fewer than five (5)

minutes.

4.1.6.8 R29

BUMPERS must be supported by the structure/frame of the ROBOT (see Figure 4-6). To be considered supported:

A. a minimum of 1 in at each end of the BUMPER must be backed by the FRAME PERIMETER,

B. the gap between the backing material and the frame must not be greater than ¼ in, and

C. no section of BUMPER greater than 8 in may be unsupported.

page 51 / 90

Figure 4-6: BUMPER Support Examples

4.1.6.9 R30

Each ROBOT must be able to display Red or Blue BUMPERS to match their ALLIANCE color, as assigned in the

MATCH schedule distributed at the event (reference Section 5.3.2).

4.1.6.10 R31

Team numbers must be displayed on the BUMPERS and meet the following criteria:

A. consist of numerals at least 4 in high, at least ½ in in stroke width, and be either white in color or outlined in

white

B. may not wrap around a corner of the FRAME PERIMETER

C. be positioned around the ROBOT such that an observer walking around the perimeter of the ROBOT can

unambiguously tell the Team’s number from any point of view.

There is no prohibition against splitting Team numbers onto different sections of

BUMPER. The intent is that the Team’s number is clearly visible and unambiguous so

that Judges, Referees, Announcers, and other Teams can easily identify competing

ROBOTS.

page 52 / 90

4.1.7 Motors & Actuators

4.1.7.1 R32

The only motors and actuators permitted on 2013 FRC ROBOTS include the following:

Table 4-1: Legal Motors

Motor Name Part Numbers Available Max Qty

Allowed

CIM FR801-001

M4-R0062-12

AM802-001A

217-2000

PM25R-44F-1005

PM25R-45F-1004

PMR25R-45F-1003

PMR25R-44F-1005

6

BaneBots M7-RS775-12

M7-RS775-18

M5-RS550-12

M5-RS550-12-B

M5-RS545-12

M5-RS540-12

M3-RS395-12

M3-RS390-12

4

AndyMark 9015 am-0912 4

Denso Throttle Control AE235100-0160 4

VEX BAG and/or mini-CIM 217-3351

217-3371

4

AndyMark PG am-2161

am-2194

3

Window Motors 262100-3030

262100-3040

Various from FIRST® Choice

2

page 53 / 90

VEX 2-wire Motor 393 276-2177 2

Snow Blower Motor am-2235 1

Electrical solenoid actuators, no greater than 1 in stroke and rated electrical

input power no greater than 10 watts (W) continuous duty at 12 volts (VDC)

Unlimited

Drive motors or fans that are part of a motor controller or COTS computing

device

Unlimited

Fans included in the 2013 Kickoff Kit, FIRST® Choice, or as a Talon motor

controller accessory

Unlimited

COTS servos with a maximum power rating of 4W each at 6VDC

Per the Servo Industry,

Servo Max Power Rating = (Stall Torque) X (No Load Speed)

Unlimited

This is the total number of each motor a Team may use on their ROBOT, not the quantity

per part number. For example, each team may use up to six (6) CIM motors on their

ROBOT, regardless of the quantity or combination of each individual part number used.

Given the extensive amount of motors allowed on the ROBOT, Teams are encouraged to

consider the total power available from the ROBOT battery during the design and build of

the ROBOT. Stalling many motors at the same time could lead to drops in ROBOT

battery voltage that will result in loss of power to core Control System components.

4.1.7.2 R33

The integral mechanical and electrical system of any motor may not be modified. Motors, servos, and electric solenoids

used on the ROBOT shall not be modified in any way, except as follows:

A. The mounting brackets and/or output shaft/interface may be modified to facilitate the physical connection of the

motor to the ROBOT and actuated part.

B. The electrical input leads may be trimmed to length as necessary.

C. The locking pins on the window motors (P/N: 262100-3030 and 262100-3040) may be removed.

D. The connector housings on the window motors (P/N: 262100-3030 and 262100-3040) may be modified to

facilitate lead connections.

E. The Integrated Encoder Module (P/N: 276-1321) may be installed on the VEX 2-wire Motor 393 (P/N

276-2177).

F. The VEX 2-wire Motor 393 (P/N: 276-2177) gears may be changed or replaced per the Supplier instructions.

The intent of this rule is to maintain the maximum power level for each ROBOT, yet still

allow teams to modify mounting tabs and the like, not to gain a weight reduction by

potentially compromising the structural integrity of any motor. The integral mechanical

and electrical system of the motor is not to be modified.

Note that for the Window motors, the gearbox is considered integral to the motor, thus the

motor may not be used without the gearbox.

page 54 / 90

4.1.8 Power Distribution

4.1.8.1 R34

The only legal source of electrical energy for the ROBOT during the competition, the ROBOT battery, is one of the

following 12VDC non-spillable lead acid batteries:

A. MK Battery (P/N: ES17-12) or

B. EnerSys (P/N: NP 18-12)

Exception: Batteries integral to and part of a COTS computing device or self-contained camera are also permitted (e.g.

laptop batteries), provided they’re only used to power the COTS computing device and any peripheral COTS USB

input devices connected to the COTS computing device and they must be securely fastened to the ROBOT.

4.1.8.2 R35

The ROBOT battery must be secured such that it will not dislodge should the ROBOT be turned over or placed in any

arbitrary orientation.

4.1.8.3 R36

Each electrical terminal on the ROBOT battery and its connection (lugs, stripped wire ends, etc.) to the 6AWG wire

must be fully insulated.

4.1.8.4 R37

Non-electrical sources of energy used by the ROBOT, (i.e., stored at the start of a MATCH), shall come only from the

following sources:

A. compressed air stored in the pneumatic system,

B. a change in the altitude of the ROBOT center of gravity, and

C. storage achieved by deformation of ROBOT parts.

4.1.8.5 R38

The ROBOT battery, the main 120-amp (120A) circuit breaker (Cooper Bussman P/N: CB185-120), and the Power

Distribution (PD) Board shall be connected as shown in Figure 4-7.

page 55 / 90

Figure 4-7: Main Power Distribution

4.1.8.6 R39

All circuits, with the exceptions of those listed in R43 and R44, must connect to, and have power sourced solely by, a

single protected 12VDC WAGO connector pair (the Load Terminals) or the 5VDC supply on the PD Board (not the M6

shanks) as shown in Figure 4-8.

4.1.8.7 R40

All wiring and electrical devices, including all Control System COMPONENTS, shall be electrically isolated from the

ROBOT frame. The ROBOT frame must not be used to carry electrical current.

R40 is checked by observing a >10k? resistance either the (+) or (-) post within the APP

connector that is attached to the PD Board and any point on the ROBOT.

The chassis for the cRIO and the Axis 206 camera have grounded enclosures. Under

R40 (and for their protection), it is required that they be electrically isolated from the

ROBOT frame when installed on the ROBOT.

4.1.8.8 R41

page 56 / 90

The 120A circuit breaker must be quickly accessible from the exterior of the ROBOT.

It is recommended that the 120A circuit breaker location be clearly and obviously labeled

so it can be easily found by ARENA staff during a MATCH. Considering the ROBOT will

also have to be powered off before being removed from the PYRAMID, Teams are

encouraged to give extra consideration to the placement of the main breaker.

4.1.8.9 R42

The PD Board and all circuit breakers must be easily visible for Inspection.

4.1.8.10 R43

The cRIO power input must be connected to the 24VDC supply terminals on the PD Board shown in Figure 4-8. With

the exception of one Solenoid Breakout Board, no other electrical load can be connected to these terminals.

Please note per R70 that, for an 8-slot cRIO, the power drawn by the Solenoid Breakout

Board may not exceed 16W. For a 4-slot cRIO, it may not exceed 21W.

4.1.8.11 R44

The wireless bridge power feed must be supplied by the 12VDC-to-5VDC converter (P/N: CLL25-24S05) connected to

the marked 12VDC supply terminals at the end of the PD Board (i.e. the terminals located between the indicator LEDs,

and not the main WAGO connectors along the sides of the PD Board) shown in Figure 4-8. No other electrical load

may be connected to these terminals.

Please reference the 2013 ROBOT Power Distribution Diagram posted on the Kit of

Parts site for wireless bridge wiring information.

page 57 / 90

Figure 4-8: Wireless Bridge, cRIO, and 5VDC Power Connections

4.1.8.12 R45

Only one wire may be connected to each WAGO connector on the PD Board.

If multi-point distribution of circuit power is needed (e.g. to provide power to three (3)

KOP breakout boards via one 20A circuit), then all incoming wires may be appropriately

spliced into the main lead, and only one lead inserted into the WAGO connector to

connect the circuit.

4.1.8.13 R46

The only circuit breakers permitted for use in the PD Board are:

A. Snap Action VB3-A Series, terminal style F57

B. Snap Action MX5-A40

4.1.8.14 R47

Each branch circuit must be protected by one and only one circuit breaker on the PD Board per Table 4-2. No other

electrical load can be connected to the breaker supplying this circuit.

Table 4-2: Branch Circuit Protection

Branch Circuit Circuit Breaker Value Quantity Allowed Per Breaker

Motor Controller Up to 40A 1

Custom Circuit Up to 40A 1

Relay Module Up to 20A 1

Digital Sidecar 20A 1

Analog/Solenoid

Breakout Board

20A 3

R47 does not prohibit the use of smaller value fuses within custom circuits for additional

protection.

4.1.8.15 R48

page 58 / 90

All active circuits shall be wired with appropriately sized insulated wire:

Table 4-3: Legal Wire Size

Application Minimum Wire Size

30 – 40A circuit 12 AWG (2.052mm)

20 – 30A circuit 14 AWG (1.628mm)

5 – 10A circuit 18 AWG (1.024mm)

Between the PD Board and the Analog and/or Solenoid

Breakout Boards (even though they are protected by a

20A circuit breaker per R47)

Between the PD Board and the cRIO 20 AWG (0.8128mm)

Between the PD Board and the wireless bridge

?5A circuit

Pneumatic valves 24 AWG (0.5106mm)

Wires that are originally attached to legal devices are part of the device and by default legal as supplies. Such wires

are exempt from R48.

4.1.8.16 R49

Branch circuits may include intermediate elements such as COTS connectors, splices, COTS flexible/rolling/sliding

contacts, and COTS slip rings, as long as the entire electrical pathway is via appropriately gauged/rated elements.

4.1.8.17 R50

All active circuit wiring with a constant polarity (i.e., except for outputs of relay modules, motor controllers, or sensor

outputs) shall be color-coded as follows:

A. Red, white, brown, or black-with-stripe on the +24VDC, +12VDC, and +5VDC connections

B. Black or blue for the common or negative side (-) of the connections.

4.1.8.18 R51

The only power regulating devices for actuators permitted on the ROBOT include:

A. Jaguar motor controller (P/N: MDL-BDC, MDL-BDC24, and 217-3367),

B. Victor 884 motor controller (P/N: VICTOR-884-12/12),

C. Victor 888 motor controller (P/N: 217-2769),

D. Talon motor controller (P/N: CTRE_Talon and am-2195),

E. VEX motor controller (P/N: 276-2193) for controlling VEX 2-wire Motor 393 (P/N: 276-2177) only, and

F. Spike H-Bridge Relay (P/N: 217-0220).

4.1.8.19 R52

page 59 / 90

Each power regulating device may control electrical loads per Table 4-4. Unless otherwise noted, each power

regulating device may control one and only one electrical load.

Table 4-4: Legal Power Regulating Device Use

Electrical Load Jaguar, Victor, or

Talon motor

controller

Spike

H-Bridge

Relay

VEX motor controller Solenoid

Breakout

am PG motor

M3-RS390-12

M3-RS395-12

M5-RS545-12

M5-RS555-12

M7-RS775-12

262100-3030

262100-3040

ARA Window motors

AE235100-0610

Yes

Up to 2 per controller

Yes No No

CIM

am-0912

M5-RS540-12

M5-RS550-12

M5-RS550-12-B

M7-RS775-18

217-3351

217-3371

Yes No No No

276-2177 Yes

Up to 2 per controller

Yes Yes No

Compressor No Yes No No

Pneumatic Solenoids No Yes* No Yes

Electric Solenoids No Yes No Yes

*Multiple low-load, pneumatic solenoid valves or lights may be connected to a single relay module. This would allow

one (1) relay module to drive multiple pneumatic actions or multiple lights. No other electrical load can be connected to

a relay module used in this manner.

4.1.8.20 R53

Servos must be directly connected to the PWM ports on the Digital Sidecar. They must not be connected to motor

controllers or relay modules.

page 60 / 90

4.1.8.21 R54

Custom circuits shall not directly alter the power pathways between the ROBOT battery, PD Board, motor controllers,

relays, motors, or other elements of the ROBOT control system (including the power pathways to other sensors or

circuits). Custom high impedance voltage monitoring or low impedance current monitoring circuitry connected to the

ROBOT’S electrical system is acceptable, if the effect on the ROBOT outputs is inconsequential.

4.1.9 Control, Command, & Signals System

4.1.9.1 R55

ROBOTS must be controlled via one (1) programmable National Instruments cRIO (P/N: cRIO-FRC or cRIO-FRCII),

with image version FRC_2013_v47.

There are no rules that prohibit co-processors, provided commands originate from the

cRIO to configure, enable, and specify all operating points for all power regulating

devices. This includes Jaguar motor controllers legally wired to the CAN-bus.

4.1.9.2 R56

One (1) D-Link wireless bridge (P/N: DAP-1522), hardware revision B, is the only permitted device for communicating

to and from the ROBOT during the MATCH.

Hardware revision A, distributed in 2011 and 2012, is not legal for 2013. Teams

participating in the Irael Regional may use hardware version Rev A or Rev B.

4.1.9.3 R57

The DAP-1522 wireless bridge must be connected to the cRIO Ethernet port 1 (either directly or via a CAT5 Ethernet

pigtail).

4.1.9.4 R58

Ethernet-connected COTS devices or custom circuits may connect to any remaining Ethernet port but must not

transmit or receive UDP packets using ports 1100-1200 with the exception of ports 1130 and 1140.

4.1.9.5 R59

page 61 / 90

Communication between the ROBOT and the OPERATOR CONSOLE is restricted as follows:

A. Network Ports:

A. TCP 1180: This port is typically used for camera data from the cRIO to the Driver Station (DS) when the

camera is connected to port 2 on the 8-slot cRIO (P/N: cRIO-FRC). This port is bidirectional.

B. TCP 1735: SmartDashboard, bidirectional

C. UDP 1130: Dashboard-to-ROBOT control data, directional

D. UDP 1140: ROBOT-to-Dashboard status data, directional

E. HTTP 80: Camera connected via switch on the ROBOT, bidirectional

F. HTTP 443: Camera connected via switch on the ROBOT, bidirectional

Teams may use these ports as they wish if they do not employ them as outlined above (i.e. TCP 1180 can be used to

pass data back and forth between the ROBOT and the DS if the Team chooses not to use the camera on port 2).

B. Bandwidth: 7 Mbits/second

The FMS Whitepaper has more details on how to check and optimize bandwidth usage.

4.1.9.6 R60

The cRIO, Driver Station software, and wireless bridge must be configured to correspond to the correct Team number,

per the procedures defined in Getting Started with the FRC Control System.

4.1.9.7 R61

All signals must originate from the OPERATOR CONSOLE and be transmitted to the ROBOT via the ARENA network.

4.1.9.8 R62

No form of wireless communication shall be used to communicate to, from, or within the ROBOT, except those required

per R56 and R61 (e.g. radio modems from previous FIRST competitions and Bluetooth devices are not permitted on

the ROBOT during competition).

4.1.9.9 R63

The wireless bridge must be mounted on the ROBOT such that the diagnostic lights are visible to ARENA personnel.

Teams are encouraged to mount the wireless bridge away from noise generating devices

page 62 / 90

such as motors.

4.1.9.10 R64

ROBOTS must use at least one (1) diagnostic ROBOT Signal Light (RSL) (P/N: 855PB-B12ME522).

Any RSL must be:

A. mounted on the ROBOT such that it is easily visible while standing three (3) ft in front of the ROBOT,

B. connected to the “RSL” supply terminals on a Digital Sidecar that is connected to an NI 9403 module in Slot 2

of the cRIO, and

C. wired for solid light operation, by placing a jumper between the “La” and “Lb” terminals on the light per Figure 4-9.

See the 2013 ROBOT Data Diagram on the KOP website and the item bulletin for

connection details.

Figure 4-9: Jumper on RSL

4.1.9.11 R65

The Driver Station software, cRIO, motor controllers, relay modules, wireless bridge, and batteries shall not be

tampered with, modified, or adjusted in any way (tampering includes drilling, cutting, machining, gluing, rewiring,

disassembling, etc.), with the following exceptions:

Please note that the Driver Station application is a separate application from the

Dashboard. The Driver Station software may not be modified, while teams are expected

to customize their Dashboard code.

A. User programmable code in the cRIO may be customized.

B. Dip switches on the cRIO may be set (applies to cRIO-FRC only).

C. Motor controllers may be calibrated as described in owner's manuals.

D. Fans may be attached to motor controllers and may be powered from the power input terminals.

E. If powering the compressor, the fuse on a Spike H-Bridge Relay may be replaced with a 20A Snap-Action

page 63 / 90

circuit breaker.

F. Wires, cables, and signal lines may be connected via the standard connection points provided on the devices.

G. Fasteners may be used to attach the device to the OPERATOR CONSOLE or ROBOT.

H. Labeling may be applied to indicate device purpose, connectivity, functional performance, etc.

I. Brake/Coast jumpers on motor controllers may be changed from their default location.

J. Limit switch jumpers may be removed from a Jaguar motor controller and a custom limit switch circuit may be

substituted.

K. If CAN-bus functionality is used, the Jaguar firmware must be updated as required by FIRST (see Rule R68-D).

L. The First Touch I/O module’s firmware may be modified.

Note that if you are using the FirstTouch I/O module as part of the OPERATOR

CONSOLE, you should not update the firmware if the manufacturer releases a new

version. The new version will wipe out the FIRST custom firmware and your FirstTouch

I/O module will no longer function with the Driver Station software. If a team does wipe

out the FIRST custom firmware, it can be restored via the most recent Driver Station

update.

M. Devices may be repaired, provided the performance and specifications of the component after the repair are

identical to those before the repair.

Please note that while repairs are permitted per the FRC Game Manual, the allowance is

independent of any manufacturer’s warranty. Teams make repairs at their own risk and

should assume that any warranty or RMA options are forfeited. Be aware that diagnosing

and repairing COMPONENTS such as these can be difficult.

4.1.9.12 R66

Neither 12VDC power nor relay module or motor controller outputs may be connected to the Analog/Solenoid Breakout

Boards or the Digital Sidecar (with the exception of the designated 12VDC input terminals).

4.1.9.13 R67

Every relay module, servo, and PWM motor controller shall be connected via PWM cable to the Digital Sidecar and be

controlled by signals provided from the cRIO via the Digital Sidecar. They shall not be controlled by signals from any

other source.

4.1.9.14 R68

Each Jaguar must be controlled with signal inputs sourced from the cRIO and passed via either a connected PWM

cable or a CAN-bus connection.

1. The Jaguar must receive signals via either a PWM cable or a CAN-bus connection. Both may not be used

simultaneously.

2. PWM configuration: If the Jaguar motor controller is controlled via PWM communications, the PWM port on the

page 64 / 90

Jaguar motor controller must be connected directly to a PWM port on the Digital Sidecar with a PWM cable.

No other device may be connected to these PWM ports. No other device may be connected to any other port

on the Jaguar motor controller with the exception of connection to the coast/brake port or the limit switch ports.

3. CAN-bus configuration: If the Jaguar motor controller is controlled via CAN-bus communications, each Jaguar

motor controller must be connected to either the cRIO or another CAN-bus device with a CAN-bus cable.

4. If the CAN-bus configuration is used, the firmware on gray Jaguar motor controllers must be updated to at least

Version 101 of the official FIRST firmware and Version 107 for black Jaguars.

As long as the CAN bus is wired legally so that the heartbeat from the cRIO is

maintained, all closed loop control features of the Jaguar motor controller may be used.

(That is, commands originating from the cRIO to configure, enable, and specify an

operating point for all Jaguar closed loop modes fit the intent of R55.)

4.1.9.15 R69

If CAN-bus communication is used, the CAN-bus must be connected to the cRIO through either the Ethernet network

connected to Port 1, Port 2, or the DB-9 RS-232 port connection.

A. Ethernet-to-CAN bridges or RS-232-to-CAN bridges (including the “black” Jaguars) may be used to connect

the CAN-bus to the cRIO.

B. Additional switches, sensor modules, custom circuits, third-party modules, etc. may also be placed on the

CAN-bus.

C. No device that interferes with, alters, or blocks communications between the cRIO and the Jaguars will be

permitted (tunneling packets for the purposes of passing them through an Ethernet-to-CAN bridge is acceptable

as the commands are not altered).

4.1.9.16 R70

Outputs from each Solenoid Breakout shall not cumulatively exceed 16W for the cRIO-FRC (8-slot) and 21W for the

cRIO-FRC II (4-slot).

4.1.9.17 R71

Control components must be configured to report the ROBOT’S battery voltage. Specifically:

A. A National Instruments 9201 analog module must be installed in slot 1 of the cRIO.

B. An Analog Breakout Board must be connected to this module.

C. A jumper must be installed in the “Power” position (two outer pins) on the Analog Breakout Board (see Figure 4-10).

D. The Analog Breakout Board must be powered from the PD Board.

page 65 / 90

Figure 4-10: Jumper for Battery Voltage Reading

4.1.9.18 R72

All outputs from sensors, custom circuits and additional electronics shall connect to only the following:

A. other custom circuits,

B. additional COTS electronics,

C. input ports on the Digital Sidecar,

D. input ports on the Analog Breakout Board,

E. the RS-232 port on the cRIO,

F. the Ethernet network connected to either Port 1 or Port 2 of the cRIO,

G. the CAN-bus if and only if all Jaguar motor controllers on the CAN-bus are wired in full compliance with R68

and R69, or

H. the sensor inputs on the Jaguar motor controller.

Custom circuits and additional electronics are allowed to utilize the Port 2 Ethernet bus

on the cRIO-FRC and/or the CAN-bus to communicate between devices. Note however,

that the ROBOT must be controlled by the cRIO (see R55). Thus, any additional devices

on the Ethernet or CAN-bus must not provide command signals that do not originate from

the cRIO.

4.1.9.19 R73

A noise filter may be wired across motor leads or PWM leads. Such filters will not be considered custom circuits and

will not be considered a violation of R54 or R72.

Acceptable signal filters must be fully insulated and must be one of the following:

A. A one microfarad (1 μF) or less non-polarized capacitor may be applied across the power leads of any motor on

your ROBOT (as close to the actual motor leads as reasonably possible).

B. A resistor may be used as a shunt load for the PWM control signal feeding a servo.

4.1.9.20 R74

page 66 / 90

Any decorations that involve broadcasting a signal to/from the ROBOT, such as remote cameras, must be approved by

FIRST (via e-mail to frcparts@usfirst.org) prior to the event and tested for communications interference at the venue.

Such devices, if reviewed and approved, are excluded from R62.

4.1.10 Pneumatics System

4.1.10.1 R75

To satisfy multiple constraints associated with safety, consistency, Inspection, and constructive innovation, no

pneumatic parts other than those explicitly permitted in Section 4.1.10 may be used on the ROBOT.

4.1.10.2 R76

All pneumatic components must be COTS pneumatic devices rated by their manufacturers for working pressure of at

least 125psi (with the exception of R78-D).

4.1.10.3 R77

All pneumatic COMPONENTS must be used in their original, unaltered condition. Exceptions are as follows:

A. tubing may be cut,

B. wiring for pneumatic devices may be modified to interface with the control system,

C. assembling and connecting pneumatic COMPONENTS using the pre-existing threads, mounting brackets,

quick-connect fittings, etc.,

D. removing the mounting pin from a pneumatic cylinder, provided the cylinder itself is not modified,

E. labeling applied to indicate device purpose, connectivity, functional performance, etc.

Do not, for example, paint, file, machine, or abrasively remove any part of a pneumatic

COMPONENT – this would cause the part to become a prohibited item. Consider

pneumatic COMPONENTS sacred.

4.1.10.4 R78

The only pneumatic system items permitted on 2013 FRC ROBOTS include the items listed below.

A. Items available in the 2013 KOP,

B. Pneumatic pressure vent plug valves functionally equivalent to those provided in the KOP,

Parker valves PV609-2 or MV709-2 are recommended.

page 67 / 90

C. Solenoid valves with a maximum ? in NPT port diameter, and a maximum Cv of 0.32,

D. Solenoid valves that are rated for a maximum working pressure that is less than 125 psi rating mandated above

are permitted, however if employed, an additional pressure relief valve must be added to the low pressure side

of the main regulator. The additional relief valve must be set to a lower pressure than the maximum pressure

rating for the solenoid valve,

E. Additional pneumatic tubing, with a maximum 0.160 in inside diameter, functionally equivalent to that provided

in the KOP,

F. Pressure transducers, pressure gauges, and connecting fittings,

G. Pressure regulators with a maximum bypass pressure of no more than 60 psi,

H. Pneumatic cylinders,

I. Pneumatic storage tanks, and

J. Compressors compliant with R80.

The following devices are not considered pneumatic devices and are not subject to

pneumatic rules (though they must satisfy all other rules):

A. a device that creates a vacuum

B. closed-loop COTS pneumatic (gas) shocks

C. air-filled (pneumatic) wheels

4.1.10.5 R79

If pneumatic COMPONENTS are used on the ROBOT, the following items are required as part of the pneumatic

system and must be connected in accordance with this section per Figure 4-11.

Figure 4-11: Pneumatic System Setup

4.1.10.6 R80

page 68 / 90

Compressed air on the ROBOT must be provided by one and only one compressor. Compressor specifications may

not exceed nominal 12VDC, 1.05 cfm flow rate. Off-board compressors must be controlled and powered by the

ROBOT.

The intent of this rule is to permit teams to take advantage of the weight savings

associated with keeping the compressor off-board. However, using the compressor

off-board of the ROBOT does NOT permit non-compliance with any other applicable

rules.

The compressor may be mounted on the ROBOT, or it may be left off the ROBOT and

used to pre-charge compressed air in storage tanks on the ROBOT prior to bringing the

ROBOT onto the FIELD.

4.1.10.7 R81

“Stored” air pressure on the ROBOT must be no greater than 120 psi. “Working” air pressure on the ROBOT must be

no greater than 60 psi. All working air must be provided through one primary adjustable pressure regulator.

Norgren regulator P/N: R07-100-RNEA recommended.

4.1.10.8 R82

Only the compressor, relief valve (P/N: 16-004-011), pressure switch, pressure vent plug valve, pressure gauge,

storage tanks, tubing, and connecting fittings may be in the high-pressure pneumatic circuit upstream from the

regulator.

4.1.10.9 R83

Pressure gauges must be placed in easily visible locations upstream and downstream of the regulator to display the

“stored” and “working” pressures.

4.1.10.10 R84

If the compressor is not included on the ROBOT (under the provisions of Rule R80), the regulator and high-pressure

gauge may be located on-board or off-board (but must be together), provided all other pneumatic rules are satisfied.

4.1.10.11 R85

If the regulator is kept off-board the ROBOT with the compressor, then only low-pressure (60 psi or less) “working” air

page 69 / 90

can be stored on the ROBOT.

4.1.10.12 R86

The relief valve must be attached directly to the compressor or attached by legal fittings connected to the compressor

output port. If using an off-board compressor, an additional relief valve must be included in the high pressure side of

the pneumatic circuit on the ROBOT.

If necessary, Teams are required to adjust the relief valve to release air at 125 psi. The

valve may or may not have been calibrated prior to being supplied to Teams.

4.1.10.13 R87

The pressure switch requirements are:

A. It must be connected to the high-pressure side of the pneumatic circuit (i.e. prior to the pressure regulator) to

sense the “stored” pressure of the circuit.

B. The two wires from the pressure switch must be connected directly to a digital input and ground pin on the

Digital Sidecar.

C. The cRIO must be programmed to sense the state of the switch and operate the relay module that powers the

compressor to prevent over-pressuring the system.

4.1.10.14 R88

The pressure vent plug valve must be:

A. connected to the pneumatic circuit such that, when manually operated, it will vent to the atmosphere to relieve

all stored pressure, and

B. placed on the ROBOT so that it is visible and easily accessible.

If the compressor is not used on the ROBOT, then an additional vent valve must be obtained and connected to the

high-pressure portion of the pneumatic circuit off board the ROBOT with the compressor (see R80).

4.1.10.15 R89

The outputs from multiple valves may not be plumbed together.

4.1.11 OPERATOR CONSOLE

page 70 / 90

4.1.11.1 R90

The Driver Station software provided on the Kit of Parts website is the only application permitted to specify and

communicate the operating mode (i.e. Autonomous/Teleop) and operating state (Enable/Disable) to the ROBOT. The

Driver Station software must be revision 12.19.12.00 or newer.

Teams are permitted to use a portable computing device of their choice (laptop computer,

PDAs, etc.) to host the Driver Station software while participating in competition

MATCHES.

4.1.11.2 R91

The OPERATOR CONSOLE must include a graphic display to present the Driver Station disgnostic information. It must

be positioned within the OPERATOR CONSOLE so that the screen display can be clearly seen during Inspection and

in a MATCH.

4.1.11.3 R92

Devices hosting the Driver Station software may only interface with the Field Management System (FMS) via the

Ethernet cable provided at the PLAYER STATION. The Ethernet port on the OPERATOR CONSOLE must be easily

and quickly accessible.

Teams are strongly encouraged to use pigtails on the Ethernet port used to connect to

the FMS. Such pigtails will reduce wear and tear on the device’s port and, with proper

strain relief employed, will protect the port from accidental jerks.

4.1.11.4 R93

The OPERATOR CONSOLE must not exceed 60 in long by 12 in deep (excluding any items that are held or worn by

the DRIVERS during the match).

There is a 54 in long by 2 in wide strip of hook-and-loop tape (“loop” side) along the

center of the PLAYER STATION support shelf that may be used to secure the

OPERATOR CONSOLE to the shelf. See Section 2.2.9 for details.

4.1.11.5 R94

page 71 / 90

Other than the system provided by the ARENA, no other form of wireless communications shall be used to

communicate to, from, or within the OPERATOR CONSOLE.

Examples of prohibited wireless systems include, but are not limited to, active wireless

network cards and Bluetooth devices. For the case of FRC, a motion sensing input device

(e.g. Microsoft Kinect) is not considered wireless communication and is allowed.

5 The Tournament

FIRST®, the FIRST® logo, FIRST® Robotics Competition, FRC ®, Coopertition®, and Gracious

Professionalism® are registered trademarks, and Sport for the Mind™ and ULTIMATE ASCENTSM are common

law trademarks, of the United States Foundation for Inspiration and Recognition of Science and Technology

(FIRST®). ©2013 FIRST. All rights reserved.

5.1 Overview

Each 2013 FIRST Robotics Competition (FRC) event is played in a tournament format. Each tournament consists of

three sets of MATCHES called “Practice MATCHES,” “Qualification MATCHES,” and “Elimination MATCHES.”

The purpose of the Practice MATCHES is to provide each Team with an opportunity to operate its ROBOT on the

FIELD prior to the start of the Qualification MATCHES.

The purpose of the Qualification MATCHES is to allow each Team to earn a seeding position that may qualify them for

participation in the Elimination MATCHES.

The purpose of the Elimination MATCHES is to determine the event Champions.

Each MATCH is conducted with approximately two (2)-minutes for set up, two minutes and fifteen seconds (2:15) of

game play, and one (1)-minute to clear the FIELD. After each MATCH, the ARENA must be cleared of ROBOTS and

OPERATOR CONSOLES from the MATCH that just ended. The ROBOTS and OPERATOR CONSOLES for the

following MATCH must be placed in position and ready to start before the start of the next MATCH. Event staff will

reset the ARENA elements during this time.

5.2 Practice Matches

5.2.1 Schedule

page 72 / 90

Practice MATCHES are played on the first day of each event. The Practice MATCH schedule is available as soon as

possible, but before Practice MATCHES start. Practice MATCHES are randomly assigned and each Team is assigned

an equal number of Practice MATCHES. If the number of Teams multiplied by number of Practice MATCHES is not

divisible by six, the FMS randomly selects some Teams to play an extra Practice MATCH.

5.2.2 Filler Line

Although Teams may not switch practice times, there will be a designated Filler Line at each event. Teams wanting

additional Practice MATCHES may join the Filler Line if the criteria below are met. Teams from the Filler Line will be

used on a first come, first served basis to fill empty spots in Practice MATCHES left by other Teams that do not report

to Queueing. The number of TEAMS in the Filler Line is dependent upon space at venues. The criteria for joining the

Filler Line are as follows:

A. ROBOTS in the Filler Line must have passed Inspection;

B. Teams must join the Filler Line with their ROBOT;

C. Teams may not work on their ROBOT while in the Filler Line;

D. Teams may not occupy more than one spot in the Filler Line; and

E. If a Team is queued up for their Practice MATCH, they may not also join the Filler Line.

5.3 Qualification Matches

5.3.1 Schedule

The Qualification MATCH schedule is made available as soon as possible, but no later than 1 hour before Qualification

MATCHES are scheduled to begin

5.3.2 MATCH Assignment

The Field Management System (FMS) assigns each Team two (2) ALLIANCE partners for each Qualification MATCH

using a predefined algorithm. The algorithm employs the following criteria, listed in order of priority:

1. Maximize time between each MATCH played for all Teams

2. Minimize the number of times a Team plays opposite any Team

3. Minimize the number of times a Team is allied with any Team

4. Minimize the use of SURROGATES

5. Provide even distribution of MATCHES played on Blue and Red ALLIANCE

All Teams are assigned the same number of Qualification MATCHES. If the number of Teams multiplied by number of

MATCHES is not divisible by six, the FMS randomly selects some Teams to play an extra MATCH. For the purpose of

seeding calculations, those Teams are designated as SURROGATES for the extra MATCH. If a Team plays a MATCH

as a SURROGATE, it is indicated on the MATCH schedule, and it is always their third Qualification MATCH. Figure 5-1

details information shown on a MATCH schedule.

The Qualification MATCH schedule indicates ALLIANCE partners and MATCH pairings. It also indicates the

page 73 / 90

ALLIANCE color assignment, “Red” or “Blue,” for each MATCH.

Figure 5-1: Sample MATCH Schedule

5.3.3 Qualification Score (QS)

Qualification Points are awarded to each Team at the completion of each Qualification MATCH and are dependent on

the final score:

Each Team on the winning ALLIANCE receives two (2) Qualification Points.

Each Team on the losing ALLIANCE receives zero (0) Qualification Points.

In the event of a tied score, all six Teams receive one (1) Qualification Point.

Exceptions to these scenarios are as follows:

A SURROGATE receives zero (0) Qualification Points.

A Team is declared a no-show if no member of the TEAM is in the ALLIANCE STATION at the start of the

MATCH; a no-show Team is disqualified from that MATCH and receives zero (0) Qualification Points.

During the Qualification MATCHES, Teams may be individually DISQUALIFIED in a MATCH. A DISQUALIFIED

(including via a RED CARD) Team receives zero (0) Qualification Points.

The total number of Qualification Points earned by a Team throughout their Qualification MATCHES is their

Qualification Score.

5.3.4 Qualification Seeding

All Teams in attendance are seeded during the Qualification MATCHES. If the number of Teams in attendance is 'n',

they are seeded '1' through 'n', with '1' being the highest seeded Team and 'n' being the lowest seeded Team.

The FMS ranks all Teams in decreasing order, using the following sorting criteria:

1st order sort Qualification Score

2nd order sort Cumulative sum of AUTO GOAL points

page 74 / 90

3rd order sort Cumulative sum of CLIMB points

4th order sort Cumulative sum of TELEOP GOAL points and FOUL points

5th order sort Random sorting by the FMS

5.4 Elimination Matches

At the end of the Qualification MATCHES, the top eight (8) seeded Teams become the ALLIANCE Leads. The top

seeded ALLIANCES are designated, in order, ALLIANCE One, ALLIANCE Two, etc., down to ALLIANCE Eight. Using

the ALLIANCE selection process described below, each Team chooses two (2) other Teams to join their ALLIANCE.

5.4.1 ALLIANCE Selection Process

Each Team chooses a student Team Representative who will proceed to the ARENA at the designated time (typically

before the lunch break on the final day of the event) to represent their Team. The Team Representative from each

ALLIANCE Lead is called the ALLIANCE CAPTAIN

The ALLIANCE selection process consists of two rounds during which each ALLIANCE CAPTAIN invites a Team

seeded below them in the standings to join their ALLIANCE. The invited Team must not already have declined an

invitation.

Round 1: In descending order (ALLIANCE One to ALLIANCE Eight), each ALLIANCE CAPTAIN invites a single Team.

The invited Team Representative steps forward and either accepts or declines the invitation.

If the Team accepts, it is moved into that ALLIANCE. If an invitation from a top eight ALLIANCE to another ALLIANCE

Lead is accepted, all lower ALLIANCE Leads are promoted one spot and the next highest seeded unselected Team

moves up to become ALLIANCE Eight.

If the Team declines, that Team is not eligible to be picked again and the ALLIANCE CAPTAIN extends another

invitation to a different Team. If an invitation from a top eight ALLIANCE to another ALLIANCE Lead is declined, the

declining Team may still invite Teams to join their ALLIANCE; however, it cannot accept invitations from other

ALLIANCES.

The process continues until ALLIANCE Eight makes a successful invitation.

Round 2: The same method is used for each ALLIANCE CAPTAIN'S second choice except the selection order is

reversed, with ALLIANCE Eight picking first and ALLIANCE One picking last. This process results in eight (8)

ALLIANCES of three (3) Teams each.

5.4.2 BACKUP TEAMS

Of the remaining eligible Teams, the highest seeded Teams (up to eight) should remain on standby and be ready to

play as a BACKUP TEAM. If a ROBOT from any ALLIANCE in an Elimination MATCH becomes inoperable the

ALLIANCE CAPTAIN may have the highest seeded BACKUP TEAM still available join the ALLIANCE. The resulting

ALLIANCE is then composed of four (4) Teams. The replaced Team remains part of the ALLIANCE for awards but

cannot play, even if their ROBOT is repaired.

Each ALLIANCE only has one opportunity to draw from the BACKUP TEAMS. If a second ROBOT from the

page 75 / 90

ALLIANCE becomes inoperable, then the ALLIANCE must play the following MATCHES with only two (2) (or even

one(1)) ROBOTS. It is in the best interest of all Teams to construct their ROBOTS to be as robust as possible to

prevent this situation.

Example: Three (3) Teams, A, B and C, form an ALLIANCE going into the Elimination

MATCHES. The highest seeded Team not on one of the eight (8) ALLIANCES is Team

D. During one of the Elimination MATCHES, Team C’s ROBOT becomes inoperable.

The ALLIANCE CAPTAIN decides to bring in Team D to replace Team C. Team C and

their ROBOT may not play in any subsequent Elimination MATCHES.

5.4.3 Elimination MATCH Bracket

The Elimination MATCHES take place on the afternoon following the completion of the Qualification MATCHES.

Elimination MATCHES are played in a bracket format as follows:

Figure 5-2: Elimination MATCH Bracket

In order to allow time between MATCHES for all ALLIANCES, the order of play is as follows:

QF1-1, QF2-1, QF3-1, QF4-1,

page 76 / 90

QF1-2, QF2-2, QF3-2, QF4-2,

QF1-3*, QF2-3*, QF3-3*, QF4-3*

Any Quarter-Final replays due to ties*

SF1-1, SF2-1, SF1-2, SF2-2, SF1-3*, SF2-3*

Any Semi-Final replays due to ties*

F-1, F-2, F-3*

Any Final replays due to ties*

(* - if required)

5.4.4 Elimination Scoring

In the Elimination MATCHES, Teams do not earn Qualification Points; they earn a Win, Loss or Tie. Within each series

of the Elimination MATCH bracket, the first ALLIANCE to win two MATCHES will advance.

In the case where the MATCH score of each ALLIANCE is equal, the tie is broken by awarding an extra point to the

ALLIANCE with (in the following order):

1. highest number of FOUL points awarded (i.e. the ALLIANCE that played the cleaner MATCH)

2. if FOUL points are equal, highest number of AUTO GOAL points

3. if AUTO GOAL points are equal, highest number of CLIMB points

If the criteria above are equal, the MATCH is a Tie and will be replayed if needed.

5.5 Tournament Rules

5.5.1 Safety and Security Rules

5.5.1.1 T01

All event attendees must wear safety glasses while in the ARENA.

5.5.1.2 T02

Wireless ROBOT control is not permitted outside the FIELD or Practice Field. ROBOTS must only be operated by

tether when outside the FIELD or Practice Field.

page 77 / 90

5.5.1.3 T03

ROBOTS must use the provided Practice Field radio for communication if operating wirelessly on the Practice Field.

5.5.1.4 T04

Teams are not allowed to set up their own 802.11a/b/g/n/ac (2.4GHz or 5GHz) wireless communication (e.g. access

points or ad-hoc networks) in the venue.

A wireless hot spot created by a cellular device would be considered an access point.

5.5.1.5 T05

No Team or Team member shall interfere or attempt to interfere with any other Team’s or FIRST’s wireless

communication. Except as expressly allowed for purposes of communicating with the Team’s own ROBOT on the

FIELD or a Practice Field, no Team or Team member shall connect or attempt to connect to any other Team’s or

FIRST’s wireless network.

Violation: Up to and including DISQUALIFICATION of the Team from the event. Legal action may also be pursued

based on applicable law.

Teams are encouraged to report suspected wireless security vulnerabilities to the FTA (if

at the event) or to FIRST via email to frcteams@usfirst.org.

5.5.2 Eligibility and Inspection

At each event, the Lead ROBOT Inspector (LRI) has final authority on the legality of any COMPONENT, MECHANISM,

or ROBOT. Inspectors may re-Inspect ROBOTS to ensure compliance with the rules.

ROBOTS are allowed to participate in scheduled Practice MATCHES prior to passing Inspection. However, the FTA,

LRI or Head Referee may determine at any time that the ROBOT is unsafe, per Section 3.2.1, and may prohibit further

participation in Practice MATCHES until the condition is corrected and the ROBOT passes Inspection.

5.5.2.1 T06

A TEAM is only allowed to participate in a MATCH and receive Qualification Points if their ROBOT has passed

Inspection. If it is discovered after the start of the MATCH that a ROBOT did not pass Inspection, the entire ALLIANCE

will receive a RED CARD for that MATCH.

page 78 / 90

Please take note of this rule. It is important that FRC Teams ensure their ALLIANCE

partners have passed Inspection. Allowing a partner that has not passed Inspection to

play puts the ALLIANCE at risk of DISQUALIFICATION. Teams should check with their

ALLIANCE partners early and help them to pass Inspection before competing.

Sending TEAM members to the ARENA without the ROBOT is considered participating in

a MATCH.

5.5.2.2 T07

Any ROBOT construction technique or element that is not in compliance with the ROBOT Rules must be rectified

before a ROBOT will be allowed to compete or continue competing. ROBOTS must fully pass Inspection before they

will be allowed to compete in Qualification or Elimination MATCHES.

5.5.2.3 T08

At the time of Inspection, the ROBOT must be presented with all MECHANISMS (including all COMPONENTS of each

MECHANISM), configurations, and decorations that will be used on the ROBOT during the entire competition event. It

is acceptable, however, for a ROBOT to play MATCHES with a subset of the MECHANISMS that were present during

Inspection. Only MECHANISMS that were present during the Inspection may be added, removed or reconfigured

between MATCHES. If MECHANISMS are changed between MATCHES, the reconfigured ROBOT must still meet all

Inspection criteria.

5.5.2.4 T09

The ROBOT Bill of Materials (BOM) must be presented at the time of Inspection.

Teams are encouraged to use the BOM Template posted on the FIRST website. Please

note that while BOMs must be shown to Inspectors, FRC Teams are not required to

submit their BOMs to the Inspectors.

5.5.2.5 T10

If a ROBOT is modified after it has passed Inspection, other than modifications described in T8, that ROBOT must be

re-Inspected.

If an observation is made that another Team’s ROBOT may be in violation of the ROBOT

rules, please approach FIRST officials to review the matter in question. This is an area

page 79 / 90

where Gracious ProfessionalismTM is very important.

5.5.2.6 T11

At events, Teams may only produce FABRICATED ITEMS in the pit areas or provided machine shops, as defined in

the Administrative Manual, Section 4.8, The Pit.

5.5.2.7 T12

For the safety of all those involved, Inspections must take place with the ROBOT powered off, pneumatics

unpressurized, and springs or other stored energy devices in their lowest potential energy states (i.e. battery removed).

Power and air pressure should only be enabled on the ROBOT during those portions of the Inspection process where it

is absolutely required to validate certain system functionality and compliance with specific rules (firmware check, etc.).

Inspectors may allow the ROBOT to be powered up beyond the parameters above if both criteria below are met.

A. The ROBOT design requires power or a charged stored energy device in order to confirm that the ROBOT

meets volume requirements, and

B. The Team has included safety interlocks that prevent unexpected release of such stored energy.

5.5.3 Referee Interaction

The Head Referee has the ultimate authority in the ARENA during the event, but may receive input from additional

sources, e.g. Game Designers, FIRST personnel, and technical staff. The Head Referee rulings are final. The Head

Referee will not review recorded replays under any circumstances.

5.5.3.1 T13

If a TEAM needs clarification on a ruling or score, one (1) pre-college student from that TEAM should address the

Head Referee after the ARENA reset signal (i.e. PLAYER STATION LED strings turn green). A TEAM member signals

their desire to speak with the Head Referee by standing in the Red or Blue Question Box, which are located on the

floor at each end of the scoring table. Depending on timing, the Head Referee may postpone any requested discussion

until the end of the subsequent MATCH.

5.5.4 YELLOW and RED CARDS

The Head Referee may assign a YELLOW or RED CARD as a result of egregious ROBOT or Team member behavior

at the ARENA.

A YELLOW or RED CARD is indicated by the Head Referee standing in front of the Team’s PLAYER STATION and

holding a YELLOW or RED CARD in the air after the completion of the MATCH.

page 80 / 90

Once a Team receives a YELLOW or RED CARD, its Team number will be colored yellow on the audience screen at

the beginning of all subsequent MATCHES as a reminder to the Team, the Referees, and the audience that they carry

a YELLOW CARD.

Examples of egregious behavior include, but are not limited to, severe and/or repeated

violations of a rule and inappropriate behavior.

A Team is issued a RED CARD (DISQUALIFICATION) in any subsequent MATCH in which they receive an additional

YELLOW CARD. This occurs after the completion of the MATCH.

A RED CARD is indicated by the Head Referee standing in front of the Team’s PLAYER STATION and holding a

YELLOW CARD and RED CARD in the air simultaneously. The Team will still carry their YELLOW CARD into

subsequent MATCHES.

YELLOW CARDS do not carry forward between Qualification MATCHES and Elimination MATCHES (i.e. all Teams

move into the Elimination MATCHES with a clean slate).

During the Elimination MATCHES, if a Team receives a RED CARD, the entire ALLIANCE is DISQUALIFIED for that

MATCH.

5.5.5 ARENA Reset Rules

5.5.5.1 T14

At the conclusion of a MATCH, TEAMS must remain in their ALLIANCE STATION until the ARENA reset signal is

issued, as indicated by the PLAYER STATION LED strings illuminating green.

5.5.5.2 T15

ROBOTS will not be re-enabled after the conclusion of the MATCH, nor will Teams be permitted to tether to the

ROBOT.

5.5.5.3 T16

If, in the judgment of the Head Referee, an “ARENA fault” occurs that affects the outcome of the MATCH, the MATCH

will be replayed. Example ARENA faults include broken FIELD elements, power failure to a portion of the FIELD,

improper activation of the FMS, errors by FIELD personnel, etc.

5.5.6 TIMEOUT and BACKUP TEAM Rules

There are no TIMEOUTS in the Qualification MATCHES. If a ROBOT cannot report for a MATCH, the Lead Queuer

page 81 / 90

should be informed and at least one member of the TEAM should report to the ARENA for the MATCH to avoid

receiving a RED CARD, with the exception of a Team that has not passed Inspection, per Rule T6.

In the Elimination MATCHES, each ALLIANCE will be allotted one (1) TIMEOUT.

During a TIMEOUT, the ARENA Timer will display the time remaining in the TIMEOUT. Both ALLIANCES will enjoy the

complete 6-minute window. If an ALLIANCE completes their repairs before the ARENA Timer expires, the ALLIANCE

CAPTAIN is encouraged to inform the Head Referee that they are ready to play. If both ALLIANCES are ready to play

before the TIMEOUT expires, the next MATCH will start.

During the Elimination MATCHES, if circumstances require an ALLIANCE to play in back-to-back MATCHES, there will

be a FIELD TIMEOUT to allow Teams to prepare for the next MATCH.

5.5.6.1 T17

If an ALLIANCE wishes to call a TIMEOUT, they must submit their TIMEOUT coupon to the Head Referee within two

(2) minutes of the ARENA reset signal preceding their MATCH.

5.5.6.2 T18

There are no cascading TIMEOUTS. If an ALLIANCE calls a TIMEOUT during a FIELD TIMEOUT, the FIELD

TIMEOUT will immediately expire and the ALLIANCE’S TIMEOUT will begin

If an ALLIANCE wishes to call a TIMEOUT during a FIELD TIMEOUT, it must still do so

within two (2) minutes of the ARENA reset signal preceding their MATCH, per Rule T17.

5.5.6.3 T19

TIMEOUTS are not transferrable between ALLIANCES.

5.5.6.4 T20

If during a TIMEOUT an ALLIANCE CAPTAIN determines that they need to call up a BACKUP ROBOT, they must

submit their BACKUP ROBOT coupon to the Head Referee while there is still at least two (2) minutes remaining on the

ARENA Timer. After that point, they will not be allowed to utilize the BACKUP ROBOT.

Alternatively, an ALLIANCE CAPTAIN may choose to call up a BACKUP ROBOT without using their TIMEOUT by

informing the Head Referee directly within two (2) minutes of the Head Referee issuing the ARENA reset signal

preceding their MATCH.

In the case where the ALLIANCE CAPTAIN’S ROBOT is replaced with the BACKUP ROBOT, the ALLIANCE

CAPTAIN is allowed in the ALLIANCE STATION as a thirteenth ALLIANCE member so they can serve in an advisory

role to their ALLIANCE.

page 82 / 90

5.5.6.5 T21

An ALLIANCE may request neither a TIMEOUT nor a BACKUP TEAM after an Elimination MATCH is stopped by the

Head Referee (e.g. due to an ARENA fault or a safety issue). The sole exception is if the replay is due to an ARENA

fault that rendered a ROBOT inoperable.

If an Elimination MATCH is replayed per T21 the Head Referee has the option of calling

a FIELD TIMEOUT.

5.5.7 Measurement

The ARENA will be open for at least one (1) hour prior to the start of Practice MATCHES, during which Teams may

survey and/or measure the FIELD. The specific time that the FIELD is open will be communicated to Teams at the

event. Teams may bring specific questions or comments to the FTA.

5.5.8 Special Equipment Rules

5.5.8.1 T22

The only equipment that may be brought in to the ALLIANCE STATION are as follows:

A. the OPERATOR CONSOLE,

B. non-powered signaling devices,

C. reasonable decorative items,

D. special clothing and/or equipment required due to a disability

E. devices used solely for the purpose of planning or tracking strategy provided they meet all of the following

conditions:

i. do not connect or attach to the OPERATOR CONSOLE

ii. do not connect or attach to the FIELD or ARENA

iii. do not connect or attach to another ALLIANCE member

iv. do not communicate with anything or anyone outside of the ARENA.

v. do not include any form of enabled wireless electronic communication (e.g. radios, walkie-talkies, cell

phones, Bluetooth communications, Wi-Fi, etc.)

vi. do not in any way affect the outcome of a MATCH, other than by allowing PLAYERS to plan or track

strategy for the purposes of communication of that strategy to other ALLIANCE members.

5.6 Championship Additions

At the 2013 FIRST Championship, Teams are split into four (4) Divisions: Archimedes, Curie, Galileo, and Newton.

Each Division plays like a Regional Event to produce the Division Champions. Those four (4) ALLIANCES proceed to

the Championship Playoffs, on the Einstein FIELD, to determine the 2013 FRC Champions.

page 83 / 90

The procedures in Sections 5.1 - 5.5 apply during the Championship, with the additions defined below.

5.6.1 Championship Pit Crews

During the Elimination MATCHES, extra Team members may be needed to maintain the ROBOT between MATCHES

because of the distance between the FIELD and the pit area. For this reason, each Team is permitted to have three (3)

additional “pit crew” members who can also help with needed ROBOT repairs/maintenance.

Only Team members wearing proper badges are allowed on the ARENA floor. FIRST will distribute these badges to

the ALLIANCE CAPTAINS during the ALLIANCE CAPTAIN meeting, which takes place on the Division FIELDS.

These badges will provide the necessary access to the ARENA for pit crew members.

Teams should assume they may be chosen for an ALLIANCE and think about the logistics of badge distribution and set

a plan prior to the pairings. It is each ALLIANCE CAPTAIN’S responsibility to get the Team’s badges to the pit crew

members.

5.6.2 Championship BACKUP ROBOT

If an ALLIANCE has not already used a BACKUP ROBOT, and an ALLIANCE ROBOT becomes inoperable during the

Championship Playoffs and cannot continue, the ALLIANCE may request a BACKUP ROBOT. The ALLIANCE

CAPTAIN will be presented the option of having one (1) of the three (3) Division Finalist ROBOTS, chosen randomly,

from their Division join the ALLIANCE as a BACKUP ROBOT.

If an ALLIANCE has won their Division with a BACKUP ROBOT and moved on to the FRC Championship Playoffs, the

BACKUP ROBOT continues to play for the ALLIANCE in the Championship Playoffs.

As noted in Section 5.4.2, the original ALLIANCE shall only have one (1) opportunity to use a BACKUP ROBOT. If the

ALLIANCE has already used a BACKUP ROBOT during the Division Elimination MATCHES or the Championship

Playoffs, they may not use a second BACKUP ROBOT. If a second ROBOT from the ALLIANCE becomes inoperable

during the Championship Playoffs, then the ALLIANCE must play the remaining MATCHES with only two (2) (or even

one(1)) ROBOTS.

5.6.3 FRC Championship MATCH Bracket

page 84 / 90

Figure 5-3: Championship Playoff MATCH Bracket

In order to allow time between MATCHES for all ALLIANCES, the order of play is as follows:

SF1-1, SF2-1, SF1-2, SF2-2, SF1-3*, SF2-3*

Any Semi-Final replays due to ties*

F-1, F-2, F-3*

Any Final replays due to ties*

(* - if required)

6 Glossary

FIRST®, the FIRST® logo, FIRST® Robotics Competition, FRC ®, Coopertition®, and Gracious

Professionalism® are registered trademarks, and Sport for the Mind™ and ULTIMATE ASCENTSM are common

law trademarks, of the United States Foundation for Inspiration and Recognition of Science and Technology

(FIRST®). ©2013 FIRST. All rights reserved.

page 85 / 90

6.1 ULTIMATE ASCENT Terms

ALLIANCE: a set of three Teams (four if counting a BACKUP ROBOT) who play ULTIMATE ASCENT together.

ALLIANCE CAPTAIN: a designated student representative from the ALLIANCE Lead.

ALLIANCE STATION: the area bounded by the ALLIANCE WALL, FEEDER STATIONS, LOW GOAL, white tape and

carpet perimeter depicted in Figure 2.3.

ALLIANCE WALL: a 6 ½ ft high, 18 ft wide, wall that defines the ends of the FIELD.

ARENA: all elements of the game infrastructure required to play ULTIMATE ASCENT: the FIELD, the ALLIANCE

STATIONS, the DISCS, and all supporting communications, ARENA control, and scorekeeping equipment.

AUTO: the first 15 seconds of the MATCH in which ROBOTS operate without direct DRIVER control.

AUTO LINE: a line, marked in Red or Blue gaffers tape, located 108 in from and parallel to the CENTER LINE.

AUTO ZONE: an area on either end of the FIELD bounded by the ALLIANCE WALL, FEEDER STATIONS, LOW

GOAL, GUARDRAILS, and the nearest AUTO LINE.

BACKUP TEAM: a Team used to replace an inoperable ROBOT on an ALLIANCE during Elimination MATCHES per

Section 5.5.6.

BUMPER: a protective assembly designed to attach to the exterior of the ROBOT and constructed as specified in

Section 4.1.6, BUMPER Rules.

BUMPER ZONE: the volume contained between two virtual horizontal planes, 2 in above the floor and 10 in above

the floor.

CENTER LINE: the line, marked in White gaffers tape, which bisects the length of the FIELD.

CLIMBING/CLIMB: the action of ascending the PYRAMID, defined in Section 3.1.5.2.

COACH: a student or adult Mentor identified as the person wearing the designated "COACH" pin or button during a

MATCH.

COMPONENT: any part in its most basic configuration, which cannot be disassembled without damaging or destroying

the part or altering its fundamental function.

COTS: a “Commercial, Off-The-Shelf” COMPONENT or MECHANISM, in its unaltered, unmodified state. A COTS

item must be a standard (i.e. not custom order) part commonly available from the VENDOR, available from a

non-Team source, and available to all Teams for purchase.

Example 1: A Team orders two ROBOT grippers from RoboHands Corp. and receives

both items. They put one in their storeroom and plan to use it later. Into the other, they

drill “lightening holes” to reduce weight. The first gripper is still classified as a COTS

item, but the second gripper is now a “custom part” as it has been modified.

Example 2: A Team obtains openly available blueprints of a drive component commonly

available from Wheels-R-Us Inc. and has local machine shop “We-Make-It, Inc.”

page 86 / 90

manufacture a copy of the part for them. The produced part is NOT a COTS item,

because it is not commonly carried as part of the standard stock of We-Make-It, Inc.

Example 3: A Team obtains openly available design drawings from a professional

publication during the pre-season, and uses them to fabricate a gearbox for their ROBOT

during the build period following Kickoff. The design drawings would be considered a

COTS item, and may be used as “raw material” to fabricate the gearbox. The finished

gearbox itself would be a FABRICATED ITEM, and not a COTS item.

Generally available software modules obtained from open sources (e.g. professional

publications, commonly used FRC community-accessible web resources, industry source

code repositories, etc.) that are not specifically affiliated with individual FRC Teams are

considered COTS items.

DISABLED: a state in which a ROBOT has been commanded by the Driver Station to deactivate all outputs.

DISC: the game piece used in ULTIMATE ASCENT.

DISQUALIFIED: the status of a TEAM, as determined by the Head Referee, in which they receive zero (0) Qualification

Points in a Qualification MATCH or receive zero (0) points in an Elimination MATCH.

DRIVER: a pre-college student TEAM member responsible for operating and controlling the ROBOT.

FABRICATED ITEM: any COMPONENT or MECHANISM that has been altered, built, cast, constructed, concocted,

created, cut, heat treated, machined, manufactured, modified, painted, produced, surface coated, or conjured partially

or completely into the final form in which it will be used on the ROBOT.

FEEDER: a pre-college TEAM member that feeds DISCS to ROBOTS.

FEEDER SLOT: an opening in a FEEDER STATION through which FEEDERS enter DISCS on to the FIELD.

FEEDER STATION: the FIELD border element containing the FEEDER SLOTS.

FIELD: the area bounded by and including the GUARDRAILS, ALLIANCE WALLS, LOW GOALS, and FEEDER

STATIONS.

FIELD TIMEOUT: a TIMEOUT called by the Head Referee.

FOUL: a penalty assessed by a Referee which credits the opponent with three (3) points.

FRAME PERIMETER: the polygon defined by the outer-most set of exterior vertices on the ROBOT (without the

BUMPERS attached) that are within the BUMPER ZONE. To determine the FRAME PERIMETER, wrap a piece of

string around the ROBOT at the level of the BUMPER ZONE - the string describes this polygon.

Note: to permit a simplified definition of the FRAME PERIMETER and encourage a tight,

robust connection between the BUMPERS and the FRAME PERIMETER, minor

protrusions such as bolt heads, fastener ends, rivets, etc are excluded from the

determination of the FRAME PERIMETER.

page 87 / 90

GOAL: a FIELD element in which DISCS are SCORED to earn points for an ALLIANCE.

GUARDRAIL: a system that borders the length of the FIELD and consists of horizontal pipes, vertical struts, and

polycarbonate shields.

HIGH GOAL: the GOAL centered over the ALLIANCE WALL.

Kit of Parts (KOP): the collection of items listed on any Kit of Parts Checklist, available in FIRST® Choice, or obtained

via a Product Donation Voucher (PDV)

LOADING ZONE: a trapezoidal area bounded by two (2) FEEDER STATIONS, an ALLIANCE WALL, a GUARDRAIL,

and Red or Blue tape.

LOW GOAL: the GOAL located between a FEEDER STATION and ALLIANCE WALL.

MATCH: a two (2) minute and fifteen (15) second period of time in which ALLIANCES compete in ULTIMATE

ASCENT.

MECHANISM: a COTS or custom assembly of COMPONENTS that provide specific functionality on the ROBOT. A

MECHANISM can be disassembled (and then reassembled) into individual COMPONENTS without damage to the

parts.

MIDDLE GOAL: a GOAL located above the ALLIANCE WALL and adjacent to the HIGH GOAL.

OPERATOR CONSOLE: the set of COMPONENTS and MECHANISMS used by the DRIVERS to relay commands to

the ROBOT.

PLAYER STATION: a position behind the ALLIANCE WALL from where a TEAM operates their ROBOT.

PLAYING CONFIGURATION: The physical configuration and orientation of the ROBOT while playing the game (i.e.

after the MATCH has started, and the ROBOT has deployed mechanisms, moved away from the starting location,

and/or interacted with the field, GAME PIECES, or other ROBOTS). This configuration is dynamic, and may change

multiple times during the course of a single MATCH.

PYRAMID: the Red or Blue steel structure on which ROBOTS CLIMB for points.

PYRAMID GOAL: a GOAL positioned on top of a PYRAMID.

RED CARD: a penalty assessed which DISQUALIFIES a TEAM, see Section 5.5.4.

ROBOT: an electromechanical assembly built by an FRC Team to perform specific tasks when competing

in ULTIMATE ASCENT. It includes all of the basic systems required to be an active participant in the game: power,

communications, control, mobility, and actuation. The implementation must obviously follow a design approach

intended to play ULTIMATE ASCENT (e.g. a box of unassembled parts placed on the FIELD or a ROBOT designed to

play a different game would not satisfy this definition).

SCORED: A DISC is considered SCORED in an ALLIANCE’S GOAL if any part of the DISC has crossed through the

opening of the GOAL, is in the GOAL at the end of the MATCH, and the DISC is not in contact with any ROBOT from

that ALLIANCE. Additionally, for a DISC to be considered SCORED in an ALLIANCE’S PYRAMID GOAL, it must

correspond to the PYRAMID color.

STARTING CONFIGURATION: The physical configuration and orientation of the ROBOT when the MATCH is started.

This is the state of the ROBOT immediately before being Enabled by the Field Management System, before the

page 88 / 90

ROBOT takes any actions, deploys any mechanisms, or moves away from the starting location. This configuration is

static, and does not change during a single MATCH (although it may change from MATCH to MATCH). In the

STARTING CONFIGURATION, no part of the ROBOT may extend outside the vertical projection of the FRAME

PERIMETER, with the exception of minor protrusions such as bolt heads, fastener ends, rivets, etc.

If a ROBOT is designed as intended and pushed up against a vertical wall (in STARTING

CONFIGURATION and with BUMPERS removed), only the FRAME PERIMETER (or

minor protrusions) will be in contact with the wall.

STARTING LINE: the line, marked in White tape, located 4 ft. behind the ALLIANCE WALL behind which TEAM

members must stay during AUTO.

SURROGATE: a Team randomly assigned by the Field Management System to play an extra Qualification MATCH,

detailed in Section 5.3.2.

TEAM: 1 COACH, 2 DRIVERS, and 1 FEEDER from the same FRC Team.

TECHNICAL FOUL: a penalty assessed by a Referee which credits the opponent with twenty (20) points.

TELEOP: the remaining two (2)-minutes of a MATCH in which ROBOTS may be directly controlled by DRIVERS.

TIMEOUT: a period of up to six (6) minutes, which an ALLIANCE can use to pause Elimination MATCH progression.

VENDOR: a legitimate business source for COTS items that satisfies all of the following criteria:

A. has a Federal Tax Identification number. In cases where the VENDOR is outside of the United States, they

must possess an equivalent form of registration or license with the government of their home nation that

establishes and validates their status as a legitimate business licensed to operate within that country.

B. is not a “wholly owned subsidiary” of an FRC Team or collection of FRC Teams. While there may be some

individuals affiliated with both an FRC Team and the VENDOR, the business and activities of the Team and

VENDOR must be completely separable.

C. must be able to ship any general (i.e., non-FIRST unique) product within five business days of receiving a valid

purchase request. It is recognized that certain unusual circumstances (such as 1,000 FIRST Teams all ordering

the same part at once from the same VENDOR) may cause atypical delays in shipping due to backorders for

even the largest VENDORs. Such delays due to higher-than-normal order rates are excused.

D. should maintain sufficient stock or production capability to fill Teams’ orders within a reasonable period during

the build season (less than 1 week). (Note that this criterion may not apply to custom-built items from a source

that is both a VENDOR and a fabricator. For example, a VENDOR may sell flexible belting that the Team

wishes to procure to use as treads on their drive system. The VENDOR cuts the belting to a custom length from

standard shelf stock that is typically available, welds it into a loop to make a tread, and ships it to a Team. The

fabrication of the tread takes the VENDOR two weeks. This would be considered a Fabricated Item, and the

two weeks ship time is acceptable.) Alternately, the Team may decide to fabricate the treads themselves. To

satisfy this criterion, the VENDOR would just have to ship a length of belting from shelf stock (i.e. a COTS item)

to the Team within five business days and leave the welding of the cuts to the Team.)

E. makes their products available to all FRC Teams. VENDOR must not limit supply or make a product available

to just a limited number of FRC Teams.

The intent of this definition it to be as inclusive as possible to permit access to all

page 89 / 90

legitimate sources, while preventing ad hoc organizations from providing special-purpose

products to a limited subset of Teams in an attempt to circumvent the cost accounting

rules. FIRST desires to permit Teams to have the broadest choice of legitimate sources

possible, and to obtain COTS items from the sources that provide them with the best

prices and level of service available. Teams also need to protect against long delays in

availability of parts that will impact their ability to complete their Robot. The FRC build

season is brief, so the Vendor must be able to get their product, particularly FIRST

unique items, to a Team in a timely manner. Ideally, chosen Vendors should have

national distributors (e.g. Home Depot, Lowes, MSC, Radio Shack, McMaster-Carr,

etc.). Remember, FRC events are not usually near home – when parts fail, local access

to replacement materials is often critical.

VISION TARGET: a retro-reflective target, made of 3M 8830 Silver Marking Film, that may be used by ROBOTS to

sense FIELD elements.

YELLOW CARD: a warning for egregious ROBOT or FRC Team member behavior.

Powered by TCPDF (www.tcpdf.org)

page 90 / 90